3GPP TSG-SA SA2 Meeting #163	S2-2406136

Jeju, Republic of Korea, 27th– 31st May 2024

Source:	Ericsson

Title:	KI#1.1-Flight Planning/Monitoring: Conclusions

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes conclusions and the way forward for KI#1.1 on pre-mission flight planning and in-mission flight monitoring in TR 23.700-59.

# 1.	Discussion

There are three solutions in TR 23.700-59 (i.e. Solutions#1, #2, and #3) addressing the KI#1 study aspect related to pre-mission flight planning and in-mission flight monitoring for UAVs (KI#1.1-Flight Planning/Monitoring).

Solution#1 proposes to enhance NEF service to assist pre-mission flight planning and in-mission flight monitoring. In detail, NEF services are enhanced to enable USS/UTM requests assistance from Core network via NEF for pre-mission flight planning and in-mission flight monitoring. Existing mechanisms are reused as much as possible for the enhancement of NEF services. For pre-mission flight planning, the existing mechanisms for obtaining list of Aerial UEs in a geographic area are used as specified in TS 23.256 [2], Movement Behaviour analytics provided by NWDAF as described in TS 23.288 [6], and the GMLC service(s) given in TS 23.273 [7] are reused. For in-mission flight monitoring, the existing mechanisms are reused, namely, UAV location reporting, UAV presence monitoring, List of aerial UEs in a geographic area as specified in TS 23.256 [2], and GLMC services for Ranging/Sidelink Positioning location results as described in TS 23.273 [7].

Solution#2 provides new NWDAF analytics on flight path deviation, which can be used for pre-mission flight planning and in-mission flight monitoring. Specifically, based on the request from USS/UTM for flight path monitoring, NEF can request analytics from NWDAF on flight path deviation. The NWDAF collects a UE location report from GMLC or MDT reports and the UE flight path report from NG RAN to derive the newly introduced analytics. If a flight path deviation is found, the NWDAF reports it to the USS/UTM via the NEF. The report also contains information on detected deviation points (e.g. 3D location) or detected deviation time or both, and confidence of the reported deviation.

Solution#3 proposes to enhance QoS Sustainability analytics from NWDAF and use it for pre-mission flight planning and in-mission flight monitoring. Specifically, USS/UTM requests notifications on QoS Sustainability analytics from the NEF for an indicated geographic area or a path of interest and in a target time interval. The NWDAF generates the QoS Sustainability Analytics considering the height information, which corresponding to the UAV flight path when deriving the likelihood of a QoS change for an Analytics target period in the future in a certain area. The analytics is used by the USS/UTM beforehand to adjust the application behaviour according to the potential QoS change.

Solution#1 provides the framework for NEF-assisted pre-mission flight planning and in-mission flight monitoring by a 3GPP network. Solutions#2 and #3 focus on the usage of analytics from NWDAF for flight path deviation monitoring and QoS based application behaviour adjustment. Therefore, Solution#1 can be the baseline and complemented with Solutions#2 and #3 aspects.

This pCR provides the conclusions for KI#1.1-Flight Planning/Monitoring.

# 2.	Text proposal

In line with the proposals made in the previous clause, it is proposed to agree the following changes to TR 23.700-59 v0.3.0:

* * * First Change * * * *

# 8	Conclusions

## 8.X	Key Issue #1: Solutions enhancing NEF services for UAS

8.X.1	KI#1.1 (NEF service enhancements)-Flight Planning/Monitoring

As this study aspect is about enhance NEF services to support service exposure for supporting pre-mission flight planning and in-mission flight monitoring for UAVs, the following principles should be considered:

-	The existing NEF services need to be reused and enhanced whenever required to introduce 5GS support for pre-mission and in-mission flight management via service exposure.

-	The introduced enhancements shall have minimum impact on other NFs, and the existing mechanisms need to be reused as much as possible.

Based on the principles, Solution#1 with necessary update can be considered as the baseline for developing a framework for NEF-assisted pre-mission flight planning and in-mission flight monitoring by a 3GPP network. However, Solution#1 can be complemented with the following aspects from other solutions: the use of flight path deviation analytics from Solution#2 and the use of QoS Sustainability analytics from Solution#3. In detail, the following aspects can be considered for the update of Solution#1 and complemented with Solutions#2 and #3.

For pre-mission flight planning:

-	Enhance the Movement Behavior Analytics by taking into consideration of the height information of UAV for the analytics generation.

-	Update the description of the NEF service to make the assistance information for pre-mission flight planning clearer, e.g. the assistance information may be the best matched route among the ones provided from the USS/UTM, or potential flight route(s) if candidate flight route(s) is not provided from the USS/UTM.

-	Add the flight path deviation analytics introduced by Solution#2 to step 9 of the procedure in clause 6.1.3.1.

-	Update the description of the procedure to make it focus on UAV while excluding UAV-C.

For in-mission flight monitoring:

-	Add new step of flight path deviation analytics introduced by Solution#2 to the procedure in clause 6.1.3.2 for UAS NF/NEF to determine whether the UAV is flight in right route.

-	Enhance the procedure for USS/UTM to subscribe to the network for UAV location periodically, and based on the collected information to determine e.g. whether UAV is flight in right route/any deviation happened.

-	Update the description of the procedure to make it focus on UAV while excluding UAV-C.

In addition, incorporate the enhanced QoS Sustainability analytics introduced by Solution#3 to the framework provided by Solution#1.

# *** End of Changes ***