3GPP TSG-SA SA2 Meeting #163	S2-2406139

Jeju, Republic of Korea, 27th– 31st May 2024

Source:	Ericsson

Title:	KI#2-NW-Assisted/Ground-based DAA: Conclusions

Document for:	Approval

Agenda Item:	9.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes conclusions and way forward for KI#2 NW-assisted/ground-based DAA in TR 23.700-59.

# 1.	Discussion

There are three solutions in TR 23.700-59 (i.e. Solutions#4, #5, and #6) address the KI#2 study on network-assisted/ground-based mechanism for DAA (Detect And Avoid) with 5GS information (KI#2-NW-Assisted/Ground-based DAA).

Solution#4 proposes a new function named Localized DAA Server (LDS) for tactical deconfliction, which is a functionality separate from USS/UTM. The LDS provides a subscription-based service available to all aerial UEs or a subset of the aerial UE, interacts with UE directly as mentioned in the step descriptions. This proposed LDS is more relevant to enabler layer, which is out the scope of core network.

Solution#5 provides UAV/UAV-C triggered Network-assisted DAA and AAM triggered Network-assisted DAA with enhanced NEF analytics exposure service. Existing mechanisms are reused as much as possible in the proposed Network-assisted DAA. In detail, the existing GMLC service on Ranging/Sidelink Positioning location and 5GC-MT-LR procedure for commercial location introduced in TS 23.273 [7] are reused, and also the Relative Proximity predictions on collision generated at NWDAF as described in TS 23.288 [6]. USS requests the services provided from 5G NFs via NEF.

Solution#6 proposes similar method as given in Soliton#5 with USS/UTM triggered the Network-assisted DAA and USS/UTM requests service from 5G NFs directly without go through NEF.

Solution#5 provides the structure of Network-assisted DAA. Solution#6 can be merged into Solution#5 after removing the content about NEF request service from 5G NFs directly. Solution#4 is out the scope of core network.

This pCR provides the conclusions for KI#2-NW-Assisted/Ground-based DAA.

# 2.	Text proposal

In line with the proposals made in the previous clause, it is proposed to agree the following changes to TR 23.700-59 v0.3.0:

* * * First Change * * * *

# 8	Conclusions

8.Y	KI#2-NW-assisted/ground-based DAA

As this key issue is about Network-assisted/ground-based DAA, and the following principles should be considered:

-	The solution provides valid method to allow Network-assisted/ground-based DAA.

-	The solution reuses as much as possible the existing 5GS information.

Based on the principles listed in above, Solution#5 with necessary update can be considered as the baseline for the structure of Network-assisted DAA. Solution#6 can be merged into Solution#5 after removing the content about NEF request service from 5G NFs directly. In detail, the following aspects can be considered for the update of Solution#5 and incorporated with Solution#6.

-	Enhance the Relative Proximity Analytics by taking consideration of the height information of UAV for the analytics generation.

-	Update the description to allow three different ways for triggering the Network-based DAA, i.e. UAV triggered, AAM triggered, and USS/UTM triggered Network-based DAA.

-	Remove the content about NEF request service from 5G NFs directly.

Solution#4 is out the scope of core network, will not be considered for normative phase.

# *** End of Changes ***