SA WG2 Meeting #163	S2-2406191

May 20 - 31, 2024 Jeju, South Korea	(Revision of S2-240xxxx)

Source: 	vivo

Title: 	KI#3 Evaluations and conclusions

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_Ph3 / Rel-19

Abstract: This contribution provides evaluation and conclusion of KI#3 Support of UE-satellite-UE communication.

# 1	Discussion

This contribution on provides the evaluation and conclusion for KI#3.

# 2 	Proposal

It is proposed to capture the following contents into TR 23.700-29.

# * * * * First change* * * *(all new text)

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.x	Key Issue #3: Support of UE-satellite-UE communication

There are 10 candidate solutions for KI#3 (Solution#28, Solution#29, Solution#30, Solution#31, Solution#32, Solution#33, Solution#40, Solution#41, Solution#42, Solution#43), which can be further categorized in the following sub-categories:

Architectures that consider ISL for the UE-satellite-UE communication link:

- Combo gNB+UPF+AGW onboard one satellite and can connect to other combo gNB+UPF+AGW onboard satellite via ISLs: #30, #32, #33,#41,#42

- Combo gNB+UPF onboard one satellite and can connect to other combo gNB+UPF onboard satellite via ISLs: #29, #31, #40

- gNB, UPF and AGW may onboard different satellite via ISLs: #32, #43

- gNB, UPF may onboard different satellite via ISLs: #28, #43

Solutions on how to solve LI is supported for the UE-satellite-UE communication link:

- AGW onboard: #32, #33, #42, #43

- AGW on the ground that P-CSCF determines to shift the UEs to the UE-satellite-UE communication when there is no need for LI, if there is a necessary, the UEs cannot be shifted to the UE-satellite-UE communication: #28, #29, #30, #31

Solutions that support the determination of UE-satellite-UE communication link:

- Same SMF determines: #29

- Same P-CSCF determines: #28, #31, #32, #40

- Different P-CSCF determines: #43

- S-CSCF determines: #30

- No need for determination, PSA UPF onboard: #33

Solutions that support handover when satellite/UE moves:

- Supporting handover: #28, #33, #40, #41, #43

- Not supporting handover or does not mention: #29, #30, #31, #32

Solutions that support roaming (LBO roaming):

- Support roaming (LBO roaming): #28, #29, #31, #32, #43

- Not supporting roaming or does not mention: #30, #33, #40, #41, #42

# * * * * Next change* * * *

# 8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.x	Key Issue #3: Support of UE-satellite-UE communication

The following interim conclusions for KI#3 " Support of UE-satellite-UE communication" are agreed:

-	2 UEs subscriptions from same HPLMN and served by same serving PLMN

NOTE 1: the roaming scenario can be standardized in next release.

-  Same SMF and same P-CSCF is used by both 2 UEs

-	The dedicated P-CSCF is used to determines whether the UE-satellite-UE communication can be performed. The same P-CSCF can ensure the determination is the same for both UEs, and lower the complexity of network.

-  The P-CSCF determines whether the UE-satellite-UE communication can be performed based on the access network info, such as satellite ID, received from UE or PCF during the call setup procedure.

NOTE 2: Two P-SCFs scenario can be standardized in next release.

-	The dedicated SMF is used to add the UPF onboarding for both UEs when determining the UE-satellite-UE communication can be performed for both UEs. The same SMF can ensure the UPF onboarding only be added when both UEs can perform UE-satellite-UE communication.

-	AGW should be on board

-	The AGW on board is used to fulfil the Lawful Interception requirement of SA3 LI

-  The PSA UPF on the ground should be selected when the IMS PDU session is established.

-	To ensure the IP address unchanged when UE moves from one satellite to another.

-	2 UEs can served by single satellite or 2 separate satellites with ISL,

-	To ensure the UE-satellite-UE communication can be continued, considering the UE will change satellite every 7 minute for LEO.

-	When one of the UEs moves to another satellite, inter-satellite handover should be performed, and the AGW should also be moved to target satellite to perform route optimization.

# * * * * End change* * * *