SA WG2 Meeting #163	S2-2406233

Jeju, Korea, May 27 – 31, 2024	(revision of S2-240xxxx)

Source:	LG Electronics

Title:	KI#1: Conclusion updates

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This paper proposes to update Conclusion for KI#1.

Discussion

TR 23.700-66 clause 8.1 includes various ENs that need to be resolved.

Proposal

It is proposed to agree the following changes into TR 23.700-66.

* * * * Start of 1st Change * * * *

8.1	Conclusion for KI#1: Network energy related information exposure

The following bullets are recommended for normative work for KI#1:

-	The following granularities of the energy consumption information are supported to be exposed, based on operator policy/configuration:

-	per Service.

-	per Network Slice level.

-	UE level.

-	PDU Session level.

-	QoS Flow (or 5QI) level.

NOTE:	whether or not to expose energy consumption per NF level or per Network Slice level by NEF will be decided in collaboration with SA5.

-	The following energy-related parameters are supported to be exposed:

-	Energy consumption for the requested Granularity of Reporting.

-	Indication that the maximum energy credit limit or the maximum energy consumption is reached/exceeded for the requested Granularity of Reporting.

-	One or more lists of top contributors that contribute the most to the energy consumption for the requested Granularity of Reporting.

-	One or more lists of contributors that ordered in descending order of contributing to the energy consumption for the requested Granularity of Reporting.

-	If all contributors contribute to the energy consumption in similar or same level, indication representing this information.

-	Remaining energy credit limit or remaining energy consumption amount for the requested Granularity of Reporting.

-	AF may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

-	The information for the calculation of the Energy Consumption information is obtained from the following sources:

-	OAM: energy related information about the network entities (e.g. energy consumption related information, energy efficiency related information of RAN nodes, 5GC NFs as described in TS 28.554 [6]).

-	UPF: data volume.

-	SMF: PDU Session related information for specific combination of DNN and S-NSSAI such as UEs, 5QI values; PDU Session related information for specific UE such as S-NSSAIs, 5QI values; QoS Flow related information for specific 5QI value such as UEs.

-	AMF: information about registered UEs such as SUPI, ULI.

-	RAN: UE-level energy consumption information.

-	A new NF is defined to support the following functionalities:

-	Collection of the information that have impact on energy consumption from other 5GC NFs and OAM.

-	Exposing the energy related information based on the request from 5GC NF or AF.

-	Calculation of energy consumption.

NOTE 1:	Exposure and collection of renewable energy and carbon emission information depend on the coordination with SA WG5.

NOTE 2:	In this Release, the gNB neither supports per-UE-per-PDU session nor per-UE-per-QoS flow level energy consumption reporting.

* * * * End of Changes * * * *