SA WG2 Meeting #163	S2-2406235

Jeju, Korea, May 27 – 31, 2024	(revision of S2-240xxxx)

Source:	LG Electronics

Title:	KI#2: Conclusion updates

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This paper proposes to update Conclusion for KI#2.

Discussion

TR 23.700-66 clause 8.2 includes various ENs that need to be resolved.

Proposal

It is proposed to agree the following changes into TR 23.700-66.

* * * * Start of 1st Change * * * *

8.2	Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria

For KI#2, the following enhancements on subscription and policy control are agreed as conclusions to support energy efficiency and energy saving as service criteria in the 5GS:

-	The following are supported:

-	Energy saving authorization information per UE to allow the network to perform energy saving for the UE.

-	Maximum energy consumption (i.e. quantity of energy for a specified period of time) is defined as energy related limits as subscription policy. It can be also provided by AF. Its granularity can be UE level, PDU Session level, QoS Flow level, etc (related to decision on granularity for KI#1).

-	The energy-related limits need to be stored in UDM/UDR.

-	The PCF may take into account energy related information including subscription data and when making policy decisions on the AM, SM and UE policies.

-	AM policy: UE-AMBR, UE-Slice-MBR, Restricted RAT type.

-	SM policy: QoS information (Session-AMBR, PDB, PER), PDU session release, PDU Session deactivation.

NOTE:	It is assumed that the controlled resources (e.g. Non-GBR QoS Flows, slices, PDU Sessions) are used only for the best-effort traffic and the guaranteed traffic is not affected.

* * * * End of Changes * * * *