SA WG2 Meeting #163	S2-2406273

Jeju, Korea, May 27-31 2024

# Title:	KI#1: conclusions

# Source:	Nokia

# Document for:	Approval

Agenda Item:	FS_EnergySys_Ph2/19.4

Abstract:	this paper provides conclusions on KI#1

1. Introduction

Based on the outcomes of the Moderated email discussion we propose to provide an update to the conclusions for KI#1

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66.

# Proposed text

## 8.1	Interim Conclusion for KI#1: Network energy related information exposure

The following bullets are recommended for normative work for KI#1:

-	The following granularities of the energy consumption information are supported to be exposed, based on operator policy/configuration:

-	per UE

-	The NF level and NS level consumption estimations may be exposed to internal functions, with source OAM. The NEF exposes at network slice level estimation to network slice customers.

-	AF may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

-	The information for the calculation of the Energy Consumption information is obtained from the following sources:

- UPF

- NG RAN

-	The granularity of information collection supported should be at UE level and if feasible for specific vendor at lower granularity.

-	The NG-RAN can detect whether a UE consume high levels of energy and report it

-	The NG-RAN can report the amount of data volume in the reporting period that was transmitted in certain energy state (defined a specific operating point in the NG-RAN: the definition of these should rely on RAN WGs to define these, but it could even include information like whether renewable energy was used).

- 	It should be possible to report vendor-specific information.

NOTE x:	additional details on how to collect energy related information from the NG-RAN and on what Energy related information from RAN to 5GC can be reported will be determined during normative work also taking into account any TSG RAN progress/feedback.

NOTE y:	Exposure and collection of renewable energy and carbon emission information may bew further discussed in normative phase in coordination with SA WG5.

NOTE z:	In this Release, the gNB neither supports per-UE-per-PDU session nor per-UE-per-QoS flow level energy consumption reporting (based on initial feedback from RAN).

-	The CHF receives the reported information from NG-RAN/UPFs (via SMF)

-	The NWDAF can receive information from CHF and via SMF from UPFs and RAN, in addition to information it can receive from OAM.

# End of proposed text