SA WG2 Meeting #163	S2-2406310

# Jeju, Korea, May 27 – May 31, 2024                         (revision of S2-24xxxxx)

Source: 	CATT

Title: 	Conclusion for KI#4

Document for: 	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_UIA_ARC / Rel-19

Abstract of the contribution: This paper is to propose some conclusions for KI#4.

2. Proposal

It is proposed to agree the following changes into TR 23.700-32.

1st Change

8.1	Key Issue #4: Identifying non-3GPP Devices Connecting behind a UE or 5G-RG

The followings are taken as conclusions:

- The Device Profile and the linked UE/5G-RG subscription identifier are provisioned to UDR by AF via NEF or are provisioned via OAM.

- PCF identifies individual non-3GPP devices connecting behind a UE or 5G-RG by using the non-3GPP Device identifier received from AF.

- PCF retrieves Device Profile of the non-3GPP device Identifier by using the SUPI of the UE/5G-RG and the non-3GPP Device identifier from the UDR.

- PCF may generate the PCC rule based on QoS information contained in the Device Profile.

- To support end-to-end QoS requirement, PCF may generate QoS parameters applied to Uu QoS flow and QoS parameters applied to non-3GPP link based on QoS information contained in the Device Profile, and provide them to SMF.

- SMF may provide QoS parameters applied to Uu QoS flow and QoS parameters applied to non-3GPP link to the UE/5G-RG.

End of Changes