SA WG2 Meeting #163	S2-2406326

Jeju, Korea, 27 - 31 May, 2024	revision of S2-240yyyy

Source:	vivo

Title:	Interim conclusion on KI#1.4

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes interim conclusion on KI#1.4.

1	Discussion

This paper proposes an interim conclusion on KI#1.4.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.3.0.

* * * * Start of Changes (all new text) * * * *

### 8.1.4	Conclusions on Key Issue #1.4

The following interim conclusions are agreed for principles of Key Issue #1.4 "Policy enhancements for DualSteer":

1)	For policy provisioning, all impacts on the AMF are optional.

2)	The DualSteer device indicates capable of steering and/or switching to the PCF transparently via AMF during registration procedure.

3)	The configuration of DualSteer traffic steering is provided by PCF based on the capabilities of the DualSteer device.

4)	The configuration of DualSteer traffic steering is not used by DualSteer device to select additional network.

5)	The DualSteer device takes into account the configuration of DualSteer traffic steering with other considerations, e.g., signalling strength, power limitation, application client request, etc., to select an access leg for establishing PDU Session.

* * * * End of Changes * * * *