SA WG2 Meeting #163	S2-2406332

Jeju, Korea, May 27 – 31, 2024	(revision of S2-2400xxxx)

Source:	vivo

Title:	KI #3, Conclusion EN Resolving

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: The paper is proposed to address ENs in conclusion for KI #3.

1. Discussion

KI #3 addresses IMS data channel interworking with MTSI UE. The contribution addresses ENs in solution for KI #3 based on discussion in SA2.

2. Proposal

It is proposed to capture the following changes into TS 23.700-77.

* * * * Start of Changes * * * *

8	Conclusions

8.1	Interim Conclusion of KI#3

For KI#3 on "IMS data channel interworking with MTSI UE" the following interim conclusions are agreed:

1.	When the originating DCMTSI UE initiates IMS DC session to the terminating MTSI UE, the originating DCSF knows the target UE does not support DC based on the SDP answer from the terminating side and determines DC interworking based on DC subscription.

2.	When the originating DCMTSI UE establishes application DC, based on the requested DC application (e.g. screen share DC application), the originating DCSF may decide to start one-side data channel procedure in this IMS session and to anchor the data channel media on the MF.

The DCSF instructs the IMS AS to create resources and association for the application data channels and how to transcode data channel media to video media for the terminating UE.

Editor's note:	The details for transmission from UE b to UE a and required transcoding is FFS.

The MF associates the data channel media resource for originating network with the video media resource for terminating network and transform data channel media to video media.

Editor's note:	The details of instruction of how to transform data channel media to video media is FFS and needs coordination with SA WG4.

3.	When the originating DCMTSI UE establishes application DC, based on the requested DC application (e.g. file share DC application), the originating DCSF may decide to indicate the DC AS to perform interworking actions for the DCMTSI UE. The DCMTSI UE can sends the shared information to DC AS and DC AS may use messaging service to send hyperlink for accessing the shared information for MTSI UE. What are the interworking actions and how messaging service is used for interworking with MTSI UE depends on DC AS implementation.

* * * * End of Changes * * * *