SA WG2 Meeting #163	S2-2406346

May 20 - 31, 2024 Jeju, South Korea	(Revision of S2-240xxxx)

Source: 	China Mobile

Title: 	KI #1, Conclusion for KI#1

Document for: 	Approval

Agenda Item: 	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: This contribution provides conclusion for key issue #1.

1	Discussion

This contribution provides the conclusion for KI#1.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-77.

FIRST CHANGE

8	Conclusions

8.X	Conclusion of KI#1

The following conclusions for KI#1 "Extensible IMS mechanism supporting IMS events in the context of DC communication" are agreed as follows:

-	The event subscription and notification is based on explicit service subscription between operator and DC AS, which specifies the scope of the notification and types of events to be notified;

-	The service subscription data for event notification is stored in IMS HSS as part of the service data, which includes the address of each DC AS subscribing events from operator network and the event types to be notified;

-	Both IMS session and DC related events should be supported and events listed in solution 7 is taken as the basis for normative phase

Session related events:

-	establishing, updating, answering, terminating of IMS session;

DC related events:

-	establishing, updating, terminating of bootstrap data channels;

-	establishing, updating, terminating of (P2A, P2A2P, P2P) application data channels;

-	downloading of an application via an existing bootstrap data channel.

-	The producer of IMS session related events is IMS AS. Whether DC related event producer is IMS AS or DCSF is decided in normative phase.

-	When the UE registers in IMS network, the IMS AS retrieves service subscription data for event notification from IMS HSS after receiving third-party registration request and knows which events are subscribed by which DC AS and the notification address of each DC AS.

-	IMS AS selects DCSF as specified in Rel-18 and notifies DCSF the event of UE registration. The DCSF retrieves service subscription data for event notification from IMS HSS and knows which events are subscribed by which DC AS and the notification address of each DC AS.

-	When the subscribed events happens, the IMS AS and DCSF notifies the events to the notification address of the DC AS.

-	If the DC AS is untrusted, the IMS AS and DCSF selects a NEF and sends the notification to NEF, who further forwards the notification to DC AS based on the received notification address.

End of CHANGES