SA WG2 Meeting #163	S2-2406359

Jeju, Korea, May 27 – 31, 2024	(revision of S2-2400xxxx)

Source:	Samsung

Title:	KI#3: Conclusion update for MTSI UE interworking

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: The paper is proposed to update the conclusion for KI #3.

1. Discussion

This contribution provides the delta part from the NWM and offline discussion for the conclusions for KI#3.

2. Proposal

It is proposed to capture the following changes into TS 23.700-77.

* * * * Start of Changes * * * *

8	Conclusions

8.1	Interim Conclusion of KI#3

For KI#3 on "IMS data channel interworking with MTSI UE" the following interim conclusions are agreed:

1.	When the originating DCMTSI UE initiates IMS DC session to the terminating MTSI UE, the originating DCSF knows the target UE does not support DC based on the SDP answer from the terminating side and determines DC interworking based on DC subscription.

2.	When the originating DCMTSI UE establishes application DC, the originating DCSF decides to start one-side data channel procedure in this IMS session and to anchor the data channel media on the MF. The DCSF instructs the IMS AS to create resources and association for the application data channels and how to transcode data channel media to video media for the terminating UE

Editor's note:	The details for transmission from UE b to UE a and required transcoding is FFS.

3.	The MF associates the data channel media resource for originating network with the video media resource for terminating network and transform data channel media to video media.

Editor's note:	The details of instruction of how to transform data channel media to video media is FFS and needs coordination with SA WG4.

4.	Alternatively, hyperlink for accessing the shared information is sent to MTSI UE. How the hyperlink is sent to the MTSI UE (e.g. whether the DC AS directly provides the hyperlink to the MTSI UE, or the DC AS provides the hyperlink to the DCMTSI UE then the DCMTSI UE sends it to the MTSI UE) is specified in normative phase. Messaging or text session may be used depends on DC application server and UE implementation.

5.	When the originating MTSI UE establishes IMS session (non-DC-session), DCAS may request to IMS AS to initiate DC session between the network and the terminating DCMTSI UE based on the event notification from the IMS AS. The DCAS provides the hyperlink for accessing the shared information to originating MTSI UE after the IMS session for originating MTSI UE and the IMS DC session for terminating DCMTSI UE are established.

* * * * End of Changes * * * *