3GPP TSG-WG SA2 Meeting #163	S2-2406366

May 27– 31, 2024, Jeju, South Korea	(revision of S2-240xxxx)

Title:	[DRAFT] Reply LS LI considerations for UE-Satellite-UE communications

Response to:	-

Release:	Release 19

Work Item:	FS_5GSAT_ARCH_Ph3

Source:	SA2

To:	SA3-LI

Contact Person:

Name:	Saubhagya Baliarsingh

E-mail Address:	saubhagya.baliarsingh@nokia.com

Attachments:

# SA2 answer:

SA2 like to thanks SA3-LI for the LS on LI considerations for UE-Satellite-UE communications.

For UE-Sat-UE scenario, SA2 has assumed both with/without AGW on-board the satellite. SA2 has assumed following concept for LI to perform intercept in the country where the warrant’s jurisdiction applies.

- The LI_X3 interface (towards AGW on-board or I-UPF on-board) for tracing media plane interface requires a feeder link connection to the serving satellite.

- The CC-TF at the P-CSCF (on the ground) will know information about the IMS-AGW(CC-POI) on board the satellite (e.g., which satellite or IP address of the AGW)

- The IRI-POI in P-CSCF will detect the target and inform the CC-TF in P-CSCF, and then CC-TF will find and send a trigger to CC-POI (in AGW) to intercept the media. P-CSCF can track the change of AGW when there is a change of serving satellite.

- Similarly, for without the AGW case, the CC-TF at the SMF (on the ground) will know information about I-UPF (CC-POI) on board the satellite. This is similar to QCI1 and QCI 5 tracing done for S8HR LI Process in 33.127, where GTP tunnels are traced based on QCI. SA2 understands, this tracing will work only for non secured SIP and RTP based IMS call.

The LI can provision interception points in all UPF or AGW in all satellites involved in UE-Sat-UE communication for interception points depending on the deployment options.

# Actions:

To SA3-LI:

ACTION: 	SA2 kindly asks SA3-LI to provide feedback on the above answers. SA2 also like to ask if it is feasible to consider UPF for IMS call interception?

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#164			19 - 23 August 2024			Maastricht, NL

3GPP SA2#165			14-18 October 2024			India