SA WG2 Meeting #163	S2-2406410

# Jeju, South Korea, May 27th – May 31st, 2024	 		(revision of S2-240)

Source: 	Nokia

Title: 	Conclusions for KI#1.1

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper provides some conclusions for KI#1.1.

# 1	Discussion

This paper proposes conclusions on KI#1.1.

# 2 Proposal

It is proposed to update TR 23.700-54 as follows:

* * * * First Change * * * *

## 8.1	Conclusions for DualSteer

### 8.1.x1	 Conclusions for KI#1.1

Below principle are agreed for KI#1.1:

1.	There is a subscription level association between the two SUPIs of a DualSteer device and it is maintained in the UDM.

2.	The Access and Mobility subscription data in UDM is enhanced to contain:

-	an indication that the SUPI is part of DualSteer;

-	the linked SUPI;

-	optionally DNN, S-NSSAI that are allowed for DualSteer switching;

4.	SMF selection subscription data sent from UDM to AMF contains information about SMF serving the PDU Session of the linked SUPI. This information is used by the AMF during PDU Session establishment for selecting a common anchor SMF for the DualSteer PDU Sessions.

3.	The Session Management subscription data is enhanced to contain the Linked SUPI and for each DNN, S-NSSAI an indication whether DualSteer switching is allowed.

* * * * End of Changes * * * *