SA WG2 Meeting #163	S2-2406413

Jeju, Korea, May 27 – 31, 2024	(revision of S2-240XXXX)

Source:	LG Electronics

Title:	KI#2 conclusion update

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract of the contribution: This paper proposes update to conclusion for KI#2 in TS 23.700-49.

Discussion

Proposes update to conclusion for KI#2.

The following ENs are resolved, respectively.

Editor's note:	Whether SMF collects N6 delay measurements from the L-PSA UPF or from the AF is FFS

Editor's note:	Details of such interaction are FFS.

- Based on the AF request, the SMF can initiate the N6 delay measurement and L-PSA UPF(s) can be instructed to measure and report N6 delay between L-PSA UPF and EAS.

Alternatively, the AF registers its available data at the NEF including the N6 delay information and provide N6 delay per candidate DNAI(s) and EAS to the 5GC based on subscription.

Editor's note:	Whether EAS load can also be used by the SMF/EASDF is FFS.

- The AF can provide the EAS load information to the 5GC, if available to be taken into account for EAS (re)selection beforehand the EAS discovery to reduce the future edge relocation triggered by the AF for load balancing with inevitable service interruption.

The SMF subscribes to the AF for EAS load information if the AF has registered its available data at the NEF with EAS load information.

Alternatively, the EASDF can be instructed by the SMF to subscribe and manage the N6 delay and/or the EAS load information locally on behalf of the SMF for less burden to the SMF and provide the information to the SMF when necessary.

Editor's note:	Whether the NWDAF should be involved is FFS.

- Considering the benefit and complexity, no additional impact on the NWDAF is expected.

Proposal

It is proposed to agree the following changes to TR 23.700-49.

* * * * Start of 1st Change * * * *

8.2	Conclusion for KI#2

The following principles are recommended in normative work for KI #2: Enhancement of EAS and local UPF (re)selection

-	SMF selects local PSA UPF and EAS considering N6 delay and/or EAS load, when available.

-	N6 delay between L-PSA UPF and EAS can be measured by leveraging existing mechanisms (e.g., defined by IETF, PING, TWAMP, OWAMP, etc.) based on the request from the AF.

-	Detailed interaction between AF and 5GC to enable the measurement will be specified in the normative.

-	Alternatively, the AF may also register its available data at the NEF including the N6 delay and/or the EAS load and provide N6 delay per candidate DNAI and/or EAS load information to the SMF/EASDF conditionally or periodically based on subscription.

-	SMF may also instruct EASDF to subscribe and manage N6 delay and/or EAS load information locally on behalf of the SMF to assist local PSA and EAS (re)selection.

* * * * End of Changes * * * *