SA WG2 Meeting #163	S2-2406456

Jeju, Korea, May 27 – May 31, 2024

Source:	vivo

Title:	KI#1: Evaluation

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution:  This pCR evaluates solutions for KI#1.

1 Discussion

In TR 23.700-84 v0.3.0, there are following solutions addressing the issue for KI#1:

- NF discovery: Sol#1, #5, #9, #10, #11;

- Data collection: Sol#1, #2, #4, #6, #7, #10, #11, #12;

- Model training: Sol#1, #2, #3, #4, #6, #7, #9, #10, #12;

- Model inference: Sol#1, #2, #3, #6, #8, #9, #10, #12;

- Accuracy monitoring: Sol#7, #8, #9;

For the similarity and difference comparing among these solutions.

2 Proposal

It is proposed to add the following changes to TR 23.700-84.

First change

7	Overall Evaluation

7.X	Key Issue #1: Enhancements to LCS to support Direct AI/ML based Positioning

The following table gives a summary for the current solutions for KI#1:

Table 7.X-1: Solutions evaluation for Key Issue#1

| Solu-tions | Aspects addressed for the Key Issue |  |  |  |  | Others |
| --- | --- | --- | --- | --- | --- | --- |
|  | NF discovery | Data collection | Model training | Model inference | Monitoring |  |
| #1 | LMF selection MTLF selection | - NWDAF trigger - from LMF - via GMLC/AMF - per area or per UE granularity |  | - NWDAF (MTLF) | - LMF (may be collocated with AnLF) |  | - Collected data could be stored in ADRF |
| #2 |  | - NWDAF/ LMF trigger - from LMF - from NWDAF |  | - NWDAF - LMF | - LMF (may be collocated with AnLF) |  |  |
| #3 |  |  |  | - NWDAF | - LMF |  |  |
| #4 |  | - NWDAF trigger - from LMF - per UE |  | - NWDAF |  |  | - Collected data could be stored in ADRF |
| #5 | LMF selection |  |  |  |  |  | - AMF authorize UE to use AI based positioning service. |
| #6 |  | - LMF trigger - may from NWDAF |  | - LMF | - LMF |  |  |
| #7 |  | - NWDAF/ LMF trigger - from AMF |  | - NWDAF - LMF |  | - NWDAF - LMF | - Collected data could be stored in ADRF - UE selection by RAN and AMF |
| #8 |  |  |  |  | - LMF | - NWDAF (MTLF) |  |
| #9 | LMF selection |  |  | - NWDAF - based on PRU data | - LMF (may be collocated with AnLF) | - LMF |  |
| #10 | MTLF selection LMF selection | - NWDAF trigger - from LMF - from AMF - from AF - via GMLC/AMF |  | - NWDAF (MTLF) | - LMF |  | - Need an Indication for requesting a ML model for Direct AI/ML positioning in ML Model request; - one or more positioning methods as ML Model filter info. |
| #11 | LMF selection | - NWDAF trigger - from LMF - Via GMLC/ AMF - LMF response to NWDAF directly. - per UE |  |  |  |  |  |
| #12 |  | - NWDAF trigger - from LMF - LMF check with UDM whether the positioning data of the UE is allowed to collect for AI positioning model training |  | - NWDAF | - LMF |  | - Add positioning data collection for AI positioning model training authorization in UE subscription data in UDM. |

Observations can be derived from the above evaluation table:

Observation 1: Many solutions (#1, #2, #3, #4, #7, #9, #10, #12) proposed that NWDAF (containing MTLF) performs AI positioning model training, while some solutions (#2, #6, #7) proposed that LMF performs AI positioning model training.

Observation 2: All solutions proposed that LMF (may standalone or collocated with AnLF) performs AI poisoning model inference.

Observation 3: NWDAF could trigger data collection from LMF (#1, #2, #4, #10, 11, #12) via GMLC/AMF (#1, #10, #11) or not (#2, #4, #12); and LMF could trigger data collection from NWDAF (#2, #6); and solution #7 proposed that NWDAF or LMF request data collection from AMF.

Observation 4: Many solutions (#1, #5, #9, #10, #11) proposed the LMF should register new capability (e.g. AI/ML based positioning related capability) to NRF for AMF to discovery and select, and some solutions (#1, #10) proposed that the NWDAF (containing MTLF) should register new capability (e.g. Supporting of Direct AI/ML positioning) to NRF for LMF to discovery and select.

Observation 5: NWDAF containing MTLF or LMF could perform accuracy monitoring for AI positioning.

End of change