3GPP TSG-SA WG2 Meeting #S2-163	                                   S2-2406462

27 MAY – 31 MAY, 2024, JeJu, Korea

Source: 	Samsung

Title: 	Interim conclusion for KI#2.2

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes interim conclusion for KI#2.2.

- Introduction

Per the NWM discussion for KI#2.2, the majority supports not having 5G NAS over non-3gpp access for simplified ATSSS. Moreover, originally this KI was intended to support data offloading to non-3GPP access without having N3IWF deployment. Accordingly Interim Conclusions for KI#2.2 are proposed.

2 Proposal

It is proposed to add the following contents to TR 23.700-54.

*** First Change(all new text) ***

8.2.2 Interim Conclusions for Key Issue #2.2

The following aspects are concluded as principles for the normative work:

- non-3GPP access without 5G NAS over non-3GPP is supported(reference solution #2.7 and #2.8):

  - During PDU Session establishment/modification procedure, UE should indicate its simplified ATSSS capability and use in the request message. And, AMF is enhanced to select appropriate SMF supporting simplified ATSSS feature.

NOTE that extension of URSP, ATSSS and MAR/N4 rules are determined in normative phase.

  - MPQUIC Steering functionality defined in TS 23.501 are reused.

Editor’s Note: Whether MPTCP steering functionality should also be supported depends on the KI#2.1 conclusion.

  - Connection between the UE and the UPF can be secured using e.g. TLS over the non-3GPP access, where the parameters for the security setup between UE and the UPF are shared via PDU Session establishment/modification message over the 3GPP access.

NOTE: details of security mechanism including UE authentication for non-3GPP access follows SA3 decision.

  - If the UE loses 3GPP access coverage, the MA PDU Session over non-3GPP access can be kept until a timer expires.

*** End of Changes ***