3GPP TSG-WG SA2 Meeting #163	S2-2406463

# Jeju, South Korea, May 27th – 31st, 2024		(revision of S2-2405177)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0363 | rev | 2 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FSA ID used for MBS session involving RedCap and non-RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Per incoming LS R2-2400078, it is mentioned that if multiple FSA IDs provide the same MBS session, then it is up to UE implementation to select the frequency according to RAN2 specification. This may cause some misunderstanding on whether the FSA ID can be differentiated per different type of UE if the MBS Session can be received both by RedCAP UEs and and non-RedCap UEs. Even if multiple FSA IDs are used for the same MBS session, there are no signaling from MB-SMF to NG-RAN on the mapping between the FSA ID and type of UE, i.e. RedCap vs non-RedCap UE. Hence the configured MBS FSA ID(s) shall not be differeneitated per type of UE. If the configured MBS FSA ID(s) is separated for RedCap UEs and non-RedCap UEs, two separated MBS sessions are to be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For one MBS session the configured MBS FSA ID(s) are applicable for both RedCap UEs and non-RedCap UEs. If the configured MBS FSA ID(s) is separated for RedCap UEs and non-RedCap UEs, two separated MBS sessions are to be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The servie may be interrupted if different MBS FSA ID is used for the same MBS session but for different type of UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.19	Support of Broadcast MBS Session Intended for NR RedCap UEs

TS 38.300 [9] defines the NR RedCap UEs that can only support limited bandwidth and have lower complexity compared to non-RedCap UEs. As defined in TS 38.331 [28], NG-RAN needs to be able to determine whether the broadcast MBS session is intended only for NR RedCap UEs, both for NR RedCap UEs and non-RedCap UEs, or only by non-RedCap UEs, in order to allocate the appropriate radio resource.

The "NR RedCap UE information" parameter comprises an indication for a broadcast MBS session that can take the following values: "MBS session expected to be received only by RedCap UEs", "MBS session expected to be received both by RedCap UEs and non-RedCap UEs", or "MBS session expected to be received only by non-RedCap UEs".

The "NR RedCap UE Information" parameter is provided by the AF to the NEF/MBSF and the MB-SMF as part of MBS Session Creation. The MB-SMF then forwards the "NR RedCap UEs Information" parameter in N2 SM Information to the NG-RAN via the AMF in MBS Session Start procedure defined in clause 7.3.1.

The "NR RedCap UE Information" parameter may be updated by the AF to the NEF/MBSF and the MB-SMF as part of MBS Session Update. The MB-SMF then forwards the updated information to the NG-RAN via the AMF in MBS Session Update procedure defined in clause 7.3.3.

NOTE:	It is up to NG-RAN implementation how the "NR RedCap UEs Information" parameter is used. When the "NR RedCap UE Information" is absent, it is up to NG-RAN implementation whether/how to enable Redcap UE to receive broadcast MBS session

If the MBS Session can be received both by RedCAP UEs and and non-RedCap UEs, the configured MBS FSA ID(s) are applicable for both RedCap UEs and non-RedCap UEs. If the configured MBS FSA ID(s) is separated for RedCap UEs and non-RedCap UEs, two separated MBS sessions are used.

# * * * * End of changes * * * *