3GPP TSG-WG SA2 Meeting #163	S2-2406554

Jeju, KR, May 27th – 31st, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2 Conclusion: For Support of Store and Forward

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_Ph3_ARCH / Rel-19

Abstract: Conclusion for KI#2.

# 1. Introduction

Based on the evaluation in S2-2406553, this pCR provides a conclusion for KI#2.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-29.

# * * * * First change (all new) * * * *

## 8.2	Conclusion for KI#2 Store and Forward

The following principles are agreed for supporting Store and Forward operation:

-	Store and forward is only supported by EPS.

-	The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC etc are on board. An implementation specific proxy is deployed on the satellite for application traffic (e.g. as described in solution #19).

-	Support of multiple satellites is required and the UE, the MME and the AF negotiate a monitoring list of satellites (e.g., only 1, all etc). The UE only listens to the satellites in the monitoring list to receive DL data/signalling from the CN (e.g. as described in solution #19).

-	The UE attaches, transfers data and detaches from each satellite as required and determined by the monitoring list.

-	The UE needs to be aware that a satellite supports S&F mode.

NOTE:	How the UE is aware that a satellite supports S&F mode of operation depends on RAN.

-	A UE may be rejected if the satellite cannot support the UE at this time and provided with a reason for the rejection and a duration until it should attempt to attach again.

# * * * * End of changes * * * *