3GPP TSG-SA WG2 Meeting #163	S2-2406572

27 May - 31 May 2024, Jeju, South Korea	(revision of S2-240xxxx)

Source:	CATT

Title:	Conclusion for KI#9: Interim Conclusion for the network information exposure

Document for:	Discussion/Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: this contribution proposes the interim conclusion for KI#9.

1.	Introduction

This contribution proposes the interim conclusion for KI#9.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First Change (all new text) * * * *

8.1	Conclusions for Key Issue#9

It is concluded that:

- The UL and or DL available data rate report can be activated for a QoS Flow by the NG-RAN based on the NGAP information from 5GC.

- The NG-RAN provides the UL and or DL available data rate report to the 5GC via the N2 interface or to the UPF via the GTP-U header of a UL packet.

- If UL and or DL available data rate report is sent to the UPF in the GTP-U header of a UL packet. UPF forwards the report to the SMF or the AF/NEF based on the N4 rules from the SMF which follows the PCC rule from the PCF.

- The report of the UL and or DL available data rate from the NG-RAN can be in different report modes (periodicity, one time, or event-based) which can be controlled by the 5GC.

- The AF can request the UL and or DL available data rate report to the 5GC during the AF requested QoS and can request a report mode for a service data flow.

- The PCF/SMF provides the UL and or DL available data rate report control information to the NG-RAN.

- If the QoS Notification Control is enabled with Alternative QoS, the NG-RAN can select different alternative QoS profiles as described in clause 5.7.2.4 of TS23.501 [2] but cannot select different alternative PDU Set QoS profiles for the packets with the same QoS/Alt-QoS indicator.

- The NG-RAN can provide a QoS Notification Control report with the selected alternative QoS profile (and including an alternative PDU Set QoS profile) Notification Control information to the AF via CP or UP based on the requests from the PCF/SMF.

- The AF can request the CP or UP mode of the QoS Notification Control report with the AF requested alternative QoS, and the PCF provides the QoS Notification Control report mode to the SMF and NG-RAN.

- If the UP mode of the QoS Notification Control report is selected by the PCF, the NG-RAN provides the QoS Notification Control report to the UPF in the GTP-U header of the UL data. UPF forwards the QoS Notification Control report to the SMF or to the AF/NEF based on the N4 rules from the SMF which follows the PCC rule from the PCF.

- Either UL/DL available data rate report or QoS Notification Control report for a QoS Flow can be activated in the NG-RAN which is determined by the PCF.

- If the alternative QoS and associated Notification Control report are used for a GBR QoS Flow, the UL and or DL available data rate report is not used for the QoS Flow.

- Since the alternative QoS cannot be used for the non-GBR QoS Flow, the UL and or DL available data rate report can be used for this non-GBR QoS Flow.

NOTE 1: The alternative PDU Set QoS can be used for the non-GBR QoS Flow, in such case, the QoS Notification Control still cannot be used for the non-GBR QoS Flow.

# * * * * End of change* * * *