SA WG2 Meeting #S2-163	S2-2406596

27 - 31 May, 2024, Jeju, South Korea	(revision of S2-24xxxxx)

Source:	OPPO

Title:	FS_MASSS update to solution #1.7 on Access Type and RAT Restriction

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS /Rel-19

Abstract of the contribution: This paper proposes update to solution #1.7 on Access Type and RAT Restriction for the FS_MASSS TR 23.700-54.

1	Discussion

This paper proposes to update solution #1.7 on Access Type Restriction. Since the SUPIs are not classified as primary SUPI or secondary SUPI, their Access Type Restriction and RAT restriction should reuse current mechanism without specific enhancement for DualSteer feature. So it is proposed to remove the Access Type/RAT restriction from the Subscription data. Another change is to reformat the subscription table to align with current subscription format in clause 5.2.3.3.1 of TS 23.502.

.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.3.0.

* * * * Start of Changes (All text new) * * * *

### 6.1.7	Solution #1.7: The UE subscription data for DualSteer in the UDM/UDR

#### 6.1.7.1	Description

This solution addresses the Key Issue #1.1: Subscription aspects to support DualSteer.

This solution shows how the UDM/UDR identifies and associates the two subscriptions/SUPIs for the DualSteer Device in the UE subscription data.

This solution is based on the following assumptions:

-	The DualSteer Device has two Ues which identified by SUPI1 and SUPI2 separately.

The following table 6.1.7.1-1 shows the example of new added “DualSteer Subscription data” in the UDM/UDR for the DualSteer Service.

Table 6.1.7.1-1 Example of DualSteer Subscription data

| Subscription data type | Field | Description |
| --- | --- | --- |
| DualSteer Subscription data | SUPI | Key. |
|  | DualSteer Authorization | Indicates whether DualSteer service is authorized for the UE. |
|  | Associated SUPI/IMSI(s) | Corresponding SUPI/IMSI(s) associated with this SUPI. The associated SUPI/IMSI can be used with the UE in the same DualSteer device for DualSteer Service. |

The above parameters in the table 6.1.7.1-1 are described as follows:

-	List of SUPIs: List of identifiers for the UE which can be identified by SUPI/IMSI.

-	DualSteer Authorization: Indicate whether the UE is authorized to use the DualSteer service.

-	Associated SUPI/IMSI: Indicate the associated UE identifier sharing a DualSteer device, which can be identified by SUPI/IMSI.

After provisioning the DualSteer Subscription data in the UE subscription data, the UDM in the 5GS can identify and associate the two subscriptions/SUPIs for the DualSteer Device if it has two single Ues.

NOTE:	This table only includes access and mobility related subscription data.

#### 6.1.7.2	Procedures

The procedure of AMF/SMF retrieving the UE subscription data from the UDM as defined in clause 4.2.2.2 and clause of 4.3.2.2 of TS 23.502 [4] can be reused.

#### 6.1.7.3	Impacts on services, entities and interfaces

The solution impacts the following network functions:

UDM/UDR:

-	Provision DualSteer Subscription data to identify and associate the subscriptions/SUPIs for DualSteer Device.

UE:

-	The USIM profile should also be configured to align with the DualSteer Subscription data in the UDM/UDR.

* * * * End of Changes * * * *