SA WG2 Meeting #S2-163	S2-2406597

27 - 31 May, 2024, Jeju, South Korea	(revision of S2-240xxxx)

Source:	OPPO

Title:	FS_MASSS Interim conclusion for KI #1.4 on policy enhancement for DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS /Rel-19

Abstract of the contribution: This paper proposes interim conclusion for KI #1.4 for the FS_MASSS TR 23.700-54.

1	Discussion

This paper proposes interim conclusion for KI #1.4 for Dual Steer.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.3.0.

* * * * Start of Changes (All text new) * * * *

8.1.X Interim Conclusions for Key Issue #1.4

The following aspects are concluded as principles for the normative work:

- Each SUPI should have its own URSP rules but the rules controlling the DualSteer feature should be the same for both SUPIs.

- URSP rule should be enhanced with Access Type preference in Route Selection Descriptor to include a Dual Steer indication.

- A new UE policy (e.g. Access Selection Policy) can be used to select the registered access network (i.e. registered UE) for traffic steering/switching. The selected UE’s URSP rule is further evaluated to associate the Application traffic to an PDU session as defined in clause 6.6.2.3 of TS 23.503[5].

- PLMN selection and RAT selection is in CT1’s scope and will not be discussed in this KI.

* * * * End of Changes * * * *