3GPP TSG-SA WG2#163	S2-2406675

Jeju, Korea, 27-31 May 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated, Vodafone

Title:	Conclusion update for KI#1: Energy related information reported from RAN node

Document for:	Discussion/Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes a conclusion for KI#1 regarding energy related information reporting from RAN node.

# 1.	Discussion

The question #1.3 was discussed in the round 1 NWM discussion as below.

Question 1.3: Which sources (e.g., NF, OAM, AF, RAN, etc.) need to support the collection of the energy related information? What granularity of the energy related information can be collected from the source?

There were 17 companies provided the responses for this question. Regarding whether RAN was needed to support the collection of the energy related information, there were 10 companies explicitly indicated that RAN may / can provide the energy information for finer granularity (e.g., UE level granularity), since RAN node is the major energy usage entity.

This contribution proposed the conclusions for KI#1 to reflect the majority companies’ opinion regarding energy related information reporting from RAN node.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-66:

# >>>>BEGINNING OF CHANGES<<<<

## 8.x	Interim Conclusion for KI#1: Network energy related information exposure

The following bullets are recommended for normative work for KI#1:

-	The following granularities of the energy consumption information are supported to be exposed, based on operator policy/configuration:

-	per Service.

-	per Network Slice level.

-	per UE level.

Editor's note:	It is FFS whether there is other granularity to be exposed is FFS

Editor's note:	whether or not to expose energy consumption per NF level or per Network Slice level by NEF is FFS.

Editor's note:	Which energy-related parameters to be exposed is FFS.

-	AF may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

-	The information for the calculation of the Energy Consumption information is obtained from the following sources:

-	NG-RAN: RAN node Energy Consumption information for UE level

Editor’s note:	How to report the Energy related information from RAN to 5GC is FFS.

Editor's note:	The sources of where the energy consumption information is obtained is FFS.

Editor's note:	What is the granularity of energy consumption information to be collected and how it is collected is FFS.

Editor's note:	How E2E energy consumption information is determined is FFS.

Editor's note:	Whether other energy related information can be exposed and how it is collected in the system is FFS.

NOTE 1:	Exposure and collection of renewable energy and carbon emission information depend on the coordination with SA WG5.

NOTE 2:	In this Release, the gNB neither supports per-UE-per-PDU session nor per-UE-per-QoS flow level energy consumption reporting.

Editor's note:	It is FFS whether the new functionality will be supported by a new and/or the existing NF(s).

Editor's note:	It is FFS whether the contribution of other NFs, e.g. SMF, AMF, will be considered.

# >>>>NEXT CHANGE<<<<

# >>>>NEXT CHANGE<<<<

# >>>>END OF CHANGES<<<<