3GPP TSG-WG SA2 Meeting #163 	S2-2406709

Jeju, Korea, May 27 – May 31, 2024

Source:	CATT

Title:	KI#1: Conclusion

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_Arch_Ph3 / Rel-19

Abstract: This contribution proposes interim conclusions for KI#1.

1. Introduction/Discussion

There are 14 candidate solutions (solutions #1, #2, #3, #4, #5, #6, #7, #8, #9, #10, #34, #35, #36, #42) for KI#1, they can be divided into two categories based on the architecture impact:

-	Cat 1: solutions without any IWK/AMF agent/Proxy over NG/S1 interface (#1, #2, #3, #4, #5, #6, #7, #8, #34, #35, and #36)

-	Cat 2: solutions with an IWK/AMF agent/Proxy over NG/S1 interface (#9, #10, and #42)

According to the LS(S2-2405600) agreed in SA2#162, the conclusion for Cat 1) is based on the following principles:

-	A procedure to handle the N2 and S1 connections when the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) should be supported. Options e.g. disconnecting/suspending/performing configuration update of the N2/S1 connections are considered in SA2. It is up to RAN3 to determine the final option about whether to reuse existing or new mechanisms/procedures.

-	The existing procedures should be re-used to support the gNB/eNB IP address change due to soft feeder link switch.

-	The AMF/MME can treat the Mapped Cell ID as per rel-17.

It is expected that the Cat. 2) is just a deployment option and has no impact on SA2/RAN’s specifications. The conclusion for Cat.2) is based on the following principle:

-	It is based on the conclusion of RAN3 whether the intermediate IWF or proxy is documented in informative Annex in TS 23.501 [2] and TS 23.401 [5] during the normative phase of the work.

2. Text Proposal

It is proposed to update TR 23.700-29 to capture the following changes.

# * * * * First change * * * *

8	Conclusions

8.2.X 	Interim conclusions for KI#1

There are two categorize the solutions:

Cat.1) Solutions which do not propose an intermediate IWF or proxy that can hide the mobility of eNB/gNB that is on board the satellite, and therefore migration of moving eNB/gNB (disconnecting and reconnecting a moving eNB/gNB leaving an area controlled by the CN) is needed

Cat. 2) Solutions which propose an intermediate IWF or proxy (called Intermediate GW for convenience) between the moving eNB/gNB and CN.

The conclusion for Cat.1) is based on the following principles:

-	A procedure to handle the N2 and S1 connections when the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) should be supported. Options e.g. disconnecting/suspending/performing configuration update of the N2/S1 connections are considered in SA2. It is up to RAN3 to determine the final option about whether to reuse existing or new mechanisms/procedures.

-	The existing procedures should be re-used to support the gNB/eNB IP address change due to soft feeder link switch.

-	The AMF/MME can treat the Mapped Cell ID as per rel-17.

It is expected that the Cat. 2) is just a deployment option and has no impact on SA2/RAN’s specifications. The conclusion for Cat.2) is based on the following principle:

-	It is based on the conclusion of RAN3 whether the intermediate IWF or proxy is documented in informative Annex in TS 23.501 [2] and TS 23.401 [5] during the normative phase of the work.

# * * * * End of changes * * * *