SA WG2 Meeting #163	S2-2406733

Jeju, South Korea , May 27-31, 2024	(was S2-240XXXX)

Source:	Samsung

Title:	KI#1: Conclusion

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes an interim conclusion for KI1.

# Proposal

It is proposed to accept the following changes to TR 23.700-77.

FIRST CHANGE

## 8.2	Conclusion for KI#1

For KI#1 on "Extensible IMS mechanism supporting IMS events in the context of DC communication" the following solution principles are proposed:

1.	The events in the SUBSCRIBE/NOTIFY framework must be extensible and shall support events that are IMS DC related and non-IMS DC related.

2.	The events in the SUBSCRIBE/NOTIFY framework must support events that are subscriber specific and support for one single IMPU or list of IMPUs. A subscriber specific event is e.g. an event like “user A has registered to the network or has established a call”, a non-subscriber event is e.g. an event like “a user has called number 800-123456” where 800-123456 is a service number.

3. The IMS AS can be discovered by HSS or NRF.

4. For HSS based discovery:

A) The IMS AS instance assigned to a UE shall register to HSS during IMS registration, and shall deregister from HSS during IMS deregistration. Existing SBI Nhss_ImsUECM service used for S-CSCF registration over N70 is enhanced to allow the registration and deregistration of the IMS AS instance in HSS over N71.

NOTE 1: 	The need to specify an Sh Diameter version of these procedures is determined during the normative phase.

B) Existing SBI Nhss_imsSubscriberDataManagement_Get service operation used for S-CSCF discovery is enhanced to allow the discovery of the IMS AS instance in HSS. The NF (e.g. AF/NEF) may use the service to discover the IMS AF instance assigned for the user.

C)	Existing SBI Nhss_imsSubscriberDataManagement_Subscribe service operation used for data change notifications is enhanced to allow the discovery of the change of the IMS AS instance HSS. The NF (e.g. AF/NEF) may use the service to discover the IMS AS instance assigned for the user has changed.

5. For NRF based discovery:

A) The IMS AS instance shall be registered with NRF with the supporting IMPU details. The NF (e.g. AF/NEF) may use NRF to get IMS AS address for the IMPU.

6.	NEF and AF may use a new Nimsas_ImsEE_xxx SBI service to manage subscriber specific and non-subscriber specific subscription requests with IMS AS(s).

NOTE 2: 	The need to manage non-subscriber specific requests with IMS AS(s) is determined during the normative phase.

7. Events supported by an event producer are discoverable via NRF.

8.	Subscription requests to IMS events shall be optimized by grouping the events into categories; event categories can be used to subscribe for a collection of individual events in an event category for example “Telephony” or “DC” event category (e.g. DC events as defined in Table AA.2.4.2.2-1 of TS 23.228 [5]).

END OF CHANGE