3GPP TSG-WG SA2 Meeting #163	S2-2406740

# Jeju, Korea, May 27- May 31, 2024

Source: 	Samsung

Title:	Update Conclusions for KI#1

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: Propose Conclusion update for KI#1.

# 1. Discussion

This proposal proposes the final conclusions for KI#1 as baseline for normative work and addresses the following EN:

Editor’s NOTE: It is FFS whether the SMF selection logic of UPF can be based on subscription/and or policy information.

In the previous meeting we already had agreement to update the Rel-18 supported UPF capabilities which are stored in NF profile or informed to SMF via N4. The NF profile only informs the SMF which particular Thus in order for the operator to control which particular functionalities are needed for a particular PDU Session, UE’s subscription data/policies received by SMF from the UDM/PCF needs to be enhanced in order to perform the UPF selection.

Otherwise it is not clear, how actually does the SMF “selects” the UPF (more specifically, UPF instance) for particular functionalities/capabilities which was the core purpose of this Key Issue.

# 2. Proposal

It is proposed to capture the following changes in TR 23.700-63.

FIRST CHANGE

# 8	Conclusions

## 8.1	Key Issue #1: Selection of UPF providing a selected user plane functionality

The following aspects are concluded as principles for normative work:

-	Reusing procedures in clause 4.17.1 and clause 4.17.2 of TS 23.502 [3] to support registering the extended UPF functionalities in NRF.

-	The following UPF functionalities are added in UPF profile stored in NRF:

-	The functionality of NAT information exposure;

-	Packet Inspection functionality (to differentiate between IP or MAC filter based packet detection, and the packet detection based on other means, e.g. layer 7 DPI);

-  Defining operator configurable parameter in the N4 capabilities and UPF NF profile which can be used for non-standard or partially supported features and configured by operator to extend the baseline UPF capabilities. The format of the parameter is to be determined by stage 3.

-	for example, the operator may configure values for this parameter to represent customized configuration (e.g, hardware accelerators (for example GPU, DPU etc.), Firewall, DDos Protection, etc.)

-	In order to assist SMF to perform UPF selection for a particular PDU Session, subscription data received from UDM can provide the information related to UPF capabilities which are needed (e.g. required/desired) for the PDU Session.

-  In order to assist SMF to perform UPF selection for a particular PDU Session, policy data received from PCF can include the information related to UPF capabilities which are needed (e.g. required or desired) for the PDU Session.

LAST CHANGE