SA WG2 Meeting #163	S2-2406756

27 - 31 May, 2024, Jeju, South Korea

Source: 	InterDigital Inc.

Title: 	KI#2, Update to Solution #10: Registration procedure for Ambient IoT Devices

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes updates to Solution#10.

# 1	Discussion

This paper proposes updates to Solution#10 Registration procedure for Ambient IoT Devices, taking into consideration additional configuration about limiting the number of AIoT devices which could be supported for registration by NG-RAN.

# 2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

## 6.10	Solution #10: Registration procedure for Ambient IoT Devices

### 6.10.1	Description

This solution is for Key Issue #2 "Identification, Subscription, Registration and Connection management".

As depicted in Architecture Requirements, the traffic types of DT and DO-DTT will be studied in this stage. The Ambient IoT devices could be driven by the network for Topology1 or UE for topology 2 before registering to the network. This proposal proposes one potential mechanism for identification, subscription, registration management and the registration procedures as well.

The principles/assumptions are given below:

-	A new network function named Ambient IoT NF may be adopted to manage Ambient IoT devices and procedures. If not, this relevant function can be supported by AMF.

-	In 5GC, each Ambient IoT device has a unique internal ID which consists of at least Operator ID, and Company. Optionally, the group ID in terms of the client or the type of item may be contained in the device ID.

-	The solution is based on an operator-controlled Ambient IoT device.

### 6.10.2	Procedures

#### 6.10.2.1	Procedures for AF triggered Registration

The following figure presents a procedure of AF triggered registration for Topology 1.

Figure 6.10.2-1 AF triggered Registration Procedure for Topology 1

0.	The Ambient IoT devices are pre-configured with default internal AIoT device ID and credentials.

The 5GC is pre-configured with the default Ambient IoT devices profile which contains the device ID and credentials too. The default internal AIoT device ID only contains Operator ID and Company info, without product info, serial number, and so on.

Editor's note:	The information contained in Operator ID is FFS.

Editor's note:	It is FFS whether it can be assumed for all scenarios that the device and CN can be pre-provisioned with CN level per device information (e.g. network layer AIoT device ID, security material).

1.	AF triggers registration and sends AF Triggered Registration Request to NEF. The service information such as the EPC code list (See GS1 TDS Release 2.1 [10]), TID code list (See GS1 TDS Release 2.1 [10]), number of devices limit, and location information are included.

Editor's note:	Clarification on how to use TID and EPC are FFS.

2.	The NEF selects an AMF or Ambient IoT NF that supports Ambient IoT services based on the location information. It will map the location information into the TA list.

3.	The NEF sends AF Triggered Registration Request to the AMF/Ambient IoT NF, including the TID list, EPC info, number of devices limit, and the TA list.

4.	The AMF/Ambient IoT NF selects NG-RAN based on the TA list.

5.	The AMF/Ambient IoT NF forwards the registration request to NG-RAN, including the TID list, EPC info, and so on.

6.	NG-RAN activates the AIoT devices based on TID list. The devices matched TID list access and register to the network via NG-RAN with EPC info. A receiving limit time may be configured on NG-RAN. Once timeout, the message received after this time will be discarded by NG-RAN. Number of devices limit if available via AF will be configured in NG-RAN, which will ensure that number of activated AIoT devices is less than or equal to the configured number of devices limit value.

7.	The AIoT devices perform interaction with 5GC for Authentication/Security. After successful registration, 5GC will generate a full internal AIoT device ID, containing the Operator ID, Company info, product info, and serial number based on EPC info.

Editor's note:	Whether to store the full internal Ambient IoT device ID with EPC info in 5GC is FFS.

8.	The NG-RAN returns the Response to the AMF/Ambient IoT NF.

9.	The AMF/Ambient IoT NF returns the response to the NEF.

10.	The NEF returns AF Triggered Registration Response to the AF.

#### 6.10.2.2	Procedures for UE triggered Registration

For Topology 2, the registration may be triggered by UE which performs as a reader. The UE interact with AMF or Ambient IoT NF via NG-RAN, which could be regarded as the supplement for AF triggered registration procedure.

Editor's note:	The procedure for UE triggered registration is FFS.

### 6.10.3	Impacts on services, entities and interfaces

NEF:

-	The NEF supports conversion between the internal AIoT device ID and EPC code.

AMF/Ambient IoT NF:

-	The AMF/Ambient IoT NF selects NG-RAN based on the TA list or gNB list and activates the AIoT devices.

UDM:

-	The UDM stores the subscription information of Ambient IoT devices in group.

NG-RAN as a reader:

-	The NG-RAN performs paging and receives response of Ambient IoT devices in Topology 1.

UE as a reader:

-	The UE performs paging and receives response of Ambient IoT devices in Topology 2.

-	The UE supports to trigger the registration procedure.

Ambient IoT device:

-	The device stores the internal AIoT device ID or the group ID.

# * * * * End of Changes * * * *