SA WG2 Meeting #163	S2-2406769

# May 27th – 31st, 2024, Jeju, South Korea

Source: 	Intel

Title: 	KI#2 conclusion update

Document for: 	Approval

Agenda Item: 	19.4 (FS_EnergySys)

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes conclusion update for KI#2 in TR 23.700-66.

# 1	Discussion

Based on the NVM discussion relevant to energy related information in making a decision for policy control and SM policy provided/updated for enforcement based on energy related information or subscription data, we propose to consider the following conclusion update for KI#2 in TR 23.700-66.

# 2 Proposal

It is proposed to include the following solution in TR 23.700-66 addressing KI#2.

* * * * First Change * * * *

## 8.2	Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria

For KI#2, the following enhancements on subscription and policy control are agreed as interim conclusions to support energy efficiency and energy saving as service criteria in the 5GS:

-	The following are supported:

-	Energy saving authorization information to allow the network to perform energy saving for the UE.

Editor's note:	It is FFS whether and what energy related limits (e.g. energy consumption limits) per UE is to define, controlled and used, which depends on whether and what energy related information per UE can be obtained as concluded in KI#1.

Editor's note:	It is FFS whether the energy saving authorization information per UE is needed.

Editor's note:	Whether the energy credit is stored in UDM/UDR or handled by the charging subsystem is FFS (decided in cooperation with SA WG5).

-	The PCF or EECF may take into account energy related information including subscription data when making or coordinating (for the case where EECF is a network functionality responsible for energy related information collection) policy decisions on the AM, SM and UE policies.

- The information in SM policy provided/updated for enforcement based on energy related information or subscription data are QoS information (Session-AMBR, PDB, PER)

- The enhancement to Session Management and UP path adjustment enhancement to consider energy related information will be defined during the normative phase.

Editor's note:	Whether the ratio of renewable energy and carbon emission information can be considered by the PCF for policy control is FFS, which depends on whether the information can be obtained as concluded in KI#1.

Editor's note:	Whether energy related information (e.g. energy consumption) per Application ID, UE, PDU Session or QoS flow can be considered by the PCF for policy control is FFS, which depends on whether the information can be obtained as concluded in KI#1.

Editor's note:	Whether the interaction with CHF for policy enforcement and charging is FFS.

* * * * End of Change * * * *