SA WG2 Meeting #163	S2-2406770

# May 27th – 31st, 2024, Jeju, South Korea

Source: 	Intel

Title: 	KI#3 conclusion update

Document for: 	Approval

Agenda Item: 	19.4 (FS_EnergySys)

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution proposes conclusion update for NWDAF analytics support for EnergySys KI#3 in TR 23.700-66.

# 1	Discussion

Based on the NVM discussion relevant to NWDAF-Based Energy Analytics for network energy saving and efficiency, we propose to consider the following conclusion update for KI#3 in TR 23.700-66.

# 2 Proposal

It is proposed to update the following conclusion in TR 23.700-66 for KI#3.

* * * * First Change * * * *

## 8.3	Conclusion for Key Issue #3: 5GS enhancements for network energy saving and efficiency

Following interim conclusions for KI#3 are proposed:

1)	For Enhancements on NF discovery and (re-)selection based on energy related information:

-	NF profile is extended (e.g. include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy.

Editor's note:	It is FFS if the information in NF profile is static information or dynamic energy related information.

-	NF discovery and (re-)selection is enhanced to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

2)  NWDAF-Based Energy Analytics for network energy saving and efficiency:

-   The analytics information from the NWDAF may be used as one of the inputs to assist NF (PCF, EECF) to make network energy saving related decision. Solution 12 is used as baseline for normative work.

NOTE: Details on the list of input parameters required to derive the analytics information and the list of output analytics will be defined in the normative phase, as this would be dependent on the conclusion outcome for KI#1/KI#2 and corresponding procedural enhancements required to support network energy saving and efficiency.

* * * * End of Change * * * *