3GPP TSG-SA WG2 Meeting #163	S2-2406793

Jeju, Korea, May 27 – May 31, 2024

Source: 	Samsung

Title: 	KI#2: Conclusions

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract of the contribution: This contribution proposes conclusions for KI#2.

# 1 Discussion

This paper proposes conclusion for KI#2.

# 2 Proposal

It is proposed to agree below proposed changes to 23.700-06.

*** Next Change ***

## 8.2	KI#2 Conclusion

It is proposed to proceed normatively based on these principles.

1)	From SA WG2 perspective there is no need to specify AS level indication in RRC connection establishment that the MWAB-UE intends to operate as MWAB.

2)	From SA WG2 perspective there is no need to indicate to NG-RAN serving a MWAB-UE that the MWAB-UE is authorized to act as MWAB-UE.

3)	The MWAB-gNB should releases the NG connections when it is no longer authorized to operate gracefully. The MWAB-gNB should hand over the UE(s) it serves to other cells before it releases the NG connection for graceful release. For the case that the BH PDU sessions are released by the MWAB-UE, the MWAB-UE does it only if the MWAB-gNB instructs the MWAB-UE that it may do so.

4)	The MWAB-UE is configured with MWAB specific configurations i.e. DNN and S-NSSAI per PLMN-ID using Registration procedure or UE configuration update procedure. If the MWAB-UE is configured with DNN and S-NSSAI for a given PLMN-ID then MWAB-UE is authorized to act as MWAB and MWAB-gNB can act as gNB for the respective PLMN-ID. The DNN and S-NSSAI per PLMN ID are stored as part subscription data in UDM

5)	The AMF receives the MWAB specific configurations i.e. DNN and S-NSSAI per PLMN-ID from UDM as part of Access and Mobility Subscription data of the UE. Before releasing the PDU sessions belonging to MWAB specific configuration, due to the existing triggers as defined in TS 23.501 or TS 23.502 or TS 24.501 for e.g. Deregistration procedure or De-authorization of MWAB operations, the AMF removes the DNN and S-NSSAI from MWAB specific configurations using UE configuration update procedure and indicate to the UE. The MWAB-UE performs graceful release of serving UEs belonging to respective PDU session as described in bullet 3 or move the serving UEs to another PDU session(based on e.g. URSP configuration) and indicate to the AMF in registration procedure that graceful release is completed. The AMF then initiates the pending NAS procedure which could release the respective PDU session.

6)	The SMF receives the MWAB specific configurations i.e. DNN and S-NSSAI from UDM as part of subscription data. Before releasing the PDU sessions belonging to MWAB specific configuration, due to the existing triggers as defined in TS 23.501 or TS 23.502 or TS 24.501, the SMF indicates to AMF that respective PDU-Session ID have to be released. Further steps of bullet 5 are executed for graceful release of serving UEs of MWAB or move the serving UEs to another PDU session (based on e.g. URSP configuration). The AMF indicates to SMF when graceful release over the respective PDU session is completed. The SMF can trigger the pending the PDU session release procedure.

7)	If MWAB-UE has trigger to perform De-registration procedure or release the PDU session belonging to the MWAB specific configurations, the MWAB-UE performs the steps specified in bullet 3 before executing the respective triggers.

8)	If the MWAB performs the actions specified in bullet 3, due to triggers specified in bullet 4,5 or 6. The MWAB-UE can trigger PDU session release procedure after executing respective actions.

9)	The MWAB-UE considers itself deauthorized for MWAB operations for a given PLMN, if it is not configured with at least one DNN and S-NSSAI for a given PLMN.