3GPP TSG-SA WG2 Meeting #163	S2-2406801

Jeju, Korea, May 27 – May 31, 2024

Source: 	Samsung

Title: 	KI#6: Conclusions

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract of the contribution: This contribution proposes conclusions for KI#6.

# 1 Discussion

This paper proposes conclusion for KI#6.

# 2 Proposal

It is proposed to agree below proposed changes to 23.700-06.

*** Next Change ***

## 8.6	KI#6 Conclusion

Emergency BH-PDU session parameters like DNN and S-NSSAI is configured in the UE using Registration procedure or UE configuration procedure.

If the serving UE triggers the emergency call, the MWAB-UE establishes the emergency BH-PDU session using the information configured in the UE e.g. URSP.

The serving UEs if they are in emergency call they can be gracefully released (if possible) following the conclusion principles of KI#2. If the serving UEs cannot be gracefully released, for e.g. they cannot be handed over to other NG-RAN then serving network should delay the pending NAS procedure, which can release the Emergency BH-PDU session.

If the MWAB-UE is on the emergency call the BH-PDU session can be released gracefully if the triggers as specified in TS 23.501, TS 24.501 or TS 23.502 occur, using the conclusion principles of KI#2.