3GPP TSG-SA WG2 Meeting #163                                                                                         S2-2406803

27 - 31 May, 2024, Jeju, Korea

Source: 	NIST

Title:	Conclusions for multi-hop UE-to-Network relay solutions

Document for:	Approval

Agenda item:	19.7

Work Item / Release: FS_5G_ProSe_Ph3 / Rel-19

Abstract of the contribution: This document proposes some conclusions related to the solutions captured in TR 23.700-03 on multi-hop UE-to-Network relay, i.e., KI#1.

# 1. 	Introduction

This contribution proposes text for the Conclusions section in TR 23.700-03, based on the common ground of Solutions #1, #2 and #7 for “KI#1: Support of multi-hop UE-to-Network Relays”.

# 2.	Text Proposal

It is proposed to agree to the following changes to 3GPP TR 23.700-03 v0.3.0.

* * * * Start of Changes – All New Text * * * *

# 8.X          Conclusions for KI#1

For Key Issue #1 (Support of multi-hop UE-to-Network Relays), the followings are taken as conclusions:

- L3 multi-hop UE-to-Network Relay can proceed into normative work.

- The service authorization and policy/parameter provisioning procedures will be described in the normative phase in coordination with other WGs.

- Standalone discovery is considered and both Model A and Model B are supported.

- The 5G ProSe Direct Discovery message will be extended with an indication that multi-hop relay is supported, along with the hop count and the maximum number of hops.

- The maximum number of hops is provisioned by the network.

- The 5G ProSe Direct Discovery message may be dropped if the maximum number of hops is reached.

- The relay path is selected based on e.g., the PC5 signal strength, number of hops between the Remote UE and the UE-to-Network Relay UE, per-hop or cumulative QoS information. The path selection criteria and the support of these metrics can be determined during normative phase and in coordination with other WGs.

- The 5G ProSe Direct Communication procedure defined in TS 23.304 is reused for the Layer-2 link establishment between each pair of UEs in the path.

- End-to-end QoS management for multi-hop U2N Relays is done similarly to the end-to-end QoS management for single hop L3 U2N Relay as defined in TS 23.304, with enhancement to handle QoS split over multiple legs of the PC5 interface.

- The QoS flows setup can be initiated by the network or the 5G ProSe Layer-3 Remote UE during Layer-2 link establishment or link modification procedure.

NOTE 1: It is left to SA WG3 to decide the details of how to support end-to-end security between the Remote UE and RAN.

* * * * End of Changes * * * *