SA WG2 Meeting #S2-163	S2-2406910

27 - 31 May, 2024, Jeju, South Korea	(Revision of S2-2405944)

Source:	Samsung, Vivo, OPPO, Tencent, Tencent Cloud, InterDigital Inc, NTT DOCOMO, ZTE, ETRI

Title:	Conclusions for KI#3

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN /Rel-19

Abstract of the contribution: This contribution proposes conclusions for KI#3.

# Discussion

Based on the discussion on NWM and documented solutions in TR 23.700-84, this contribution proposes the conclusions for KI#3.

# Proposal

It is proposed to adopt the following changes into TR 23.700-84.

*** Start of the change ***

## 8.x	Conclusion for KI#3: NWDAF-assisted policy control and QoS enhancement

Following conclusions are proposed for KI#3:

Based on PCF request, NWDAF may provide assistance information analytics to PCF to assist with QoS determination and/or modification of QoS parameters. The PCF may take the assistance information into account to determine the final QoS to be implemented, based on the operator's policy.

After the PCF derives the final QoS, based on updated assistance information provided by NWDAF (the updated assistance information may include candidate QoS parameters, the analytics of the duration and usage of established QoS Flows), the PCF may determine to provision modified QoS towards the SMF.

In detail,

- The PCF may send a single analytics request to NWDAF for QoS and policy assistance information analytics. The PCF may include analytics ID(s), Filter Information, one or multiple sets of QoS parameters, one or multiple time window(s), target service experience (e.g. QoE).

NOTE 1:	details of the input parameters in the PCF request will be discussed in normative stage.

- Based on the PCF request, the QoS and policy assistance information analytics provided by NWDAF may include:

- Expected QoE and applicable time window(s) for each of the candidate QoS parameters provided by the PCF.

NOTE 2:	the applicable time window(s) are associated with QoS parameters and are subset of the Analytics target period.

Editor’s note:	whether to introduce new service or reuse existing service for providing candidate QoS is FFS.

- Analytics that are valid and used within the same validity period, i.e Observed service experience, QoS sustainability, Network Performance analytics, analytics of the duration and usage of the established QoS Flows. Possible enhancements to the existing analytics and/or introducing the analytics will be determined in the normative phase.

NOTE 3:	To provide the assistance for policy control and QoS, NWDAF shall not be aware of PCF internal logic.

NOTE 4:	Details of the input data and corresponding sources for generating QoS assistance information and providing the corresponding analytics by NWDAF will be discussed in normative stage.

NOTE 5: NWDAF can monitor the accuracy performance of the assistance information analytics based on a request from PCF. Whether to reuse the existing procedures for Analytics Accuracy Monitoring (defined in clause 6.2D of TS 23.288 [5]) or not can be decided in the normative phase.

*** End of the change ***