SA WG2 Meeting #163	S2-2406967

Jeju, South Korea , May 27-31, 2024	(was S2-2406936)

Source:	Ericsson, Nokia, AT&T, NTT DOCOMO, Samsung

Title:	KI#1: Conclusion on solution principles

Document for:	Discussion/Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2

Abstract of the contribution: This contribution proposes an interim conclusion for KI1.

# Proposal

It is proposed to accept the following changes to TR 23.700-77.

FIRST CHANGE

## 8.x	Conclusions of KI#1

For KI#1 on "Extensible IMS mechanism supporting IMS events in the context of DC communication" the following solution principles are proposed:

1.	A new NEF service, based on SUBSCRIBE/NOTIFY requests, is defined for the IMS events in the context of DC communication.

2.	The NEF service must be extensible and shall support events that are IMS DC related and non-IMS DC related.

3.	Subscriber specific and non-subscriber specific must be supported. A subscriber specific event is e.g. an event like “user A has registered to the network or has established a call”, a non-subscriber event is e.g. an event like “a user has called number 800-123456” where 800-123456 is a service number.

A subscriber specific request includes the targeted subscribers. The targeted subscriber(s) can be either one distinct subscriber, or multiple subscribers given as a list of distinct subscribers or a wildcarded IMS user identity. The events in the request are common for all targeted subscribers. This request is proxied to the HSS instance(s) serving the given subscribers.

4.	HSS shall support the handling of subscriber specific subscribe requests. To that effect, the subscribe requests that are subscriber specific are sent to HSS which handles registered and unregistered UEs and ensures that a subscription is persistent when UE registers, deregisters and re-registers in IMS. That is, the HSS stores the subscription request and sends the subscription request to the IMS AS whenever the IMS AS instance supporting the requested event is assigned to the UE. The information about the subscription events stored in HSS is only relevant and consumed by HSS. NEF and AF use a new Nhss_ImsEE_xxx SBI service to manage subscription requests (e.g. subscribe/unsubscribe) via HSS for subscriber specific events.

5.	The IMS AS instance assigned to a UE during IMS registration shall be registered in HSS. Existing SBI Nhss_ImsUECM service used for S-CSCF registration over N70 is enhanced to allow the registration of the IMS AS instance in HSS over N71.

6.	Subscription requests that are non-subscriber specific are sent directly to all IMS AS instances.

7.	For the purpose of optimization and following 5GC exposure principles, any Notification related to a subscription is sent directly to the NF (e.g. NEF or AF) that triggered the subscription request using Nimsas_SessionEventControl Notify service operation.

NOTE 2: 	The need to specify an Sh Diameter version of these procedures is determined during the normative phase.

8.	Optionally, events supported by an event producer (i.e. IMS AS ) can be discovered via NRF.

NOTE 3: 	Events to be exposed are defined in normative phase.

NOTE 4:	Events like application download is not supported in Release 19 but the event “ADC establishment for a specific application” is supported.

9.	Subscription requests to IMS events may be optimized; event categories shall be defined to group a collection of individual events in an event category. The categories are defined during the normative phase.

NOTE 5 : In this Release only IMS AS supporting IMS DC registers its instance in the HSS.

END OF CHANGES