SA WG2 Meeting #163	S2-2406972

Jeju, Korea, May 27 – 31, 2024	(revision of S2-2406942)

Source:	ZTE

Title:	KI #3, Conclusion EN Resolving

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract of the contribution: The paper is proposed to address ENs in conclusion for KI #3.

1. Discussion

KI #3 addresses IMS data channel interworking with MTSI UE. The contribution addresses ENs in solution for KI #3 based on discussion in SA2.

2. Proposal

It is proposed to capture the following changes into TS 23.700-77.

* * * * Start of Changes * * * *

8	Conclusions

8.1	Interim Conclusion of KI#3

For KI#3 on "IMS data channel interworking with MTSI UE" the following interim conclusions are agreed:

- When the originating DCMTSI UE initiates an IMS DC session to the terminating MTSI UE, the originating DCSF is aware that the target UE does not support IMS DC based on the SDP answer from the terminating side and determines that IMS DC interworking is required for the requested DC application(s).

- For interworking with MTSI UE, the DC AS based on KI#1 is notified about the event that an application DC for an application that requires interworking is established. The DC AS may take action to provide DC application specific information to the MTSI UE by means outside the scope of 3GPP.

NOTE 1:	How the DCSF can determine that IMS DC interworking for the requested DC application(s) is required is up to implementation. DCSF can for example use the binding information to determine whether interworking is requested.

3.	IMS DC interworking is not a subscription based service.

4.	When the originating DCMTSI UE establishes an application DC, the originating DCSF terminates the data channel media at the MF. The DCSF instructs the IMS AS how to transcode the received data channel media at the MF to MMTEL media, if the received data channel media can be transcoded to MMTEL media. The transcoded MMTEL media are sent by the MF to the terminating UE.

NOTE 2:	The transcoding between MMTEL media and DC media is specified by SA4.

* * * * End of Changes * * * *