3GPP TSG-WG SA2 Meeting #163	S2-2407079

# Jeju, KR, May 27- May 31, 2024

Source: 	China Mobile, SK Telecom, Nokia, ETRI, DISH Network, Samsung, Deutsche Telekom

Title:	Conclusion update for KI#1

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: Update conclusions for KI#1.

1. Discussion

This proposal updates the conclusions for KI#1 as baseline for normative work.

2. Proposal

It is proposed to capture the following changes in TR 23.700-63.

FIRST CHANGE

8.1	Conclusions for Key Issue #1: Selection of UPF providing a selected user plane functionality

The following aspects are concluded as principles for normative work:

-	Reusing procedures in clauses 4.17.1 and 4.17.2 of TS 23.502 [3] to support registering the extended UPF functionalities in NRF.

-	The following UPF functionalities are added in the N4 capabilities and UPF NF profile stored in NRF:

-	The functionality of NAT information exposure;

-	Packet Inspection functionality (to differentiate between IP or MAC filter based packet detection, and the packet detection based on other means, e.g. layer 7 DPI);

-	Defining operator configurable parameter in the N4 capabilities and UPF NF profile which can be used for non-standard or partially supported features and configured by operator to extend the baseline UPF capabilities. The format of the parameter is to be determined by stage 3.

-	For example, the operator may configure values for this parameter to represent customized configuration (e.g. hardware accelerators (for example GPU, DPU etc.), Firewall, DDos Protection, etc.).

-	The SMF may determine the PSA UPF functionalities for PDU Session based on:

-	Subscription information from the UDM/UDR: The SMF may receive subscription information from the UDM, during (or after) the establishment of the PDU session, indicating functionalities that are required and/or preferred for the PDU session.

NOTE: The details of subscription information should be discussed in normative phase.

# * * * * End of changes * * * *