3GPP TSG-SA WG2#163	S2-2407084

Jeju, Korea, May 27 – May 31, 2024             	(was S2-2406822, S2-2405272, S2-2404154)

Source:	Apple

Title:	Conclusion for KI#2.1

Document for:	Discussion/Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes conclusion for KI#2.1.

# 1.	Introduction

This contribution proposes conclusion for KI#2.1.

NWM discussion outcome has the following proposals:

Proposal 1: more discussion is needed how to define MPQUIC steering functionalities and whether to support CONNECT-TCP.

Proposal 2: proceed with the assumption that CONNECT-Ethernet is supported in Rel-19 for Ethernet PDU Sessions.

Proposal 3: proceed with the assumption that if multiple CONNECT methods are supported, the SMF may indicate to the UE the CONNECT methods that are supported for the respective PDU Session, and then UE determines the CONNECT method to be used based on nature of traffic.

Proposal 4: proceed with the assumption that MPQUIC Connect-IP steering functionality is not to be used with Ethernet MA PDU Sessions.

# 2.	Proposal

For Proposal 1, it is proposed that new steering functionalities are defined as a new Steering Functionality (i.e. multiple MPQUIC steering functionalities, each one associated with a specific proxy mode).

It is proposed to agree the following changes vs. TR 23.700-54 v0.3.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.2	Conclusions for ATSSS_Ph4

### 8.2.1	Conclusion for Key Issue #2.1

a) 	A new MPQUIC steering functionality is defined:

-	Multipath QUIC-IP (MPQUIC-IP) steering functionality based on CONNECT-IP [7] to tunnel IP packets between a client (UE) and a proxy (UPF) using HTTP/3 over MPQUIC.

NOTE 1:  The MPQUIC steering functionality defined in Rel-18 TS 23.501 [3] refers to a Multipath QUIC-UDP (MPQUIC-UDP) steering functionality based on CONNECT-UDP [6].

NOTE 2:  The MPQUIC steering functionality can be selected if additional features, which are not supported by the ATSSS-LL steering functionality and PMF, are required for the traffic steering/switching/splitting of a service data flow.

b) 	During MA PDU Session Establishment with an IP-based PDU Session type, the UE indicates to SMF what MPQUIC steering functionalities it supports. In a single MA PDU Session, different proxy protocols may be used for different service data flows. If the UE supports multiple MPQUIC steering functionalities, it shall use the provisioned ATSSS rules to decide which steering functionality to apply for a service data flow.

c)	Additional parameters associated with a connect protocol, e.g. "ipproto" and "target" parameters associated with CONNECT-IP [7] is derived by the UE based on the detected application.

d)	MPQUIC-IP steering functionality shall not be used with Ethernet MA PDU Sessions.

# >>>>END OF CHANGES<<<<