3GPP SA WG2 Meeting #163	S2-2407164

Jeju, Korea, 27 – 31 May 2024

Source:	Nokia, NTT DOCOMO, CATT, China Mobile, Vivo

Title:	Conclusions for KI#3 for UE-Sat-UE

Document for:	Approval

Agenda Item:	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: This paper proposes interim conclusions and way forward for KI#1 in TR 23.700-29

# Discussion

Proposes conclusions for KI#3, based on proposed solution in 23.700-29 and recently concluded NWM observations.

# Proposal

*** Start of changes ***

# 8	Conclusions

## 8.x 	Key issues #3: Support of UE-Sat-UE communication

The following bullet points summarize the principles for the way forward to support UE-Sat-UE communication in regenerative architecture:

- Only IMS voice/video services are considered in this release.

- Both UEs are served by the same HPLMN.

- A ground UPF is selected and acts as an PSA UPF during the IMS PDU session, and the onboard UPF is inserted as UL-CL (local PSA) for UE-satellite-UE communication procedure during the call setup procedures.

- During call setup, P-CSCF will determine the possibility of UE-Sat-UE call based on, offeror’s satellite ID and access network information.

- P-CSCF shall provision the AGW based on the above information received from PCF and UE.

- When no AGW is onboard, e.g. an IP over IP tunnel over N6 between the onboard local UPF on MO UE side and the onboard local UPF in the MT UE side can be supported. This is up to operator’s preference to use the AGW on the ground or select the UPF onboard satellite for an IMS call, when AGW are not placed on-board satellite.

- An AGW relocation procedure is needed for change of satellite to route the UE-satellite-UE traffic to the new AGW. The simultaneous UL/CL change due to satellite change can be coordinated with AGW relocation to avoid packet loss.

NOTE 1: The scenario of UPF onboard only should be confirmed by SA3-LI

NOTE 2: The AGW relocation solution will be supported if SA3-LI confirms that is acceptable and possible.

NOTE 3: The above assumes a call involving more than one Satellite.

*** End of changes ***