3GPP TSG-WG SA2 Meeting #163	S2-2407283

# Jeju, Korea, May 27 – May 31, 2024      												 was_S2-2407067

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1296 | rev | 3 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

X

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spending Limits for UE Policies in Roaming scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle, China Telecom, Nokia, Ericsson. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_SLUPiR |  |  |  |  |  | Date: |  |  | 2024-05-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some key operators are interested in enhancing UE policies functionality of R18 SLAMUP, such that (based on a proper SLA) it will be applicable also in the roaming scenario (both LBO and HR). For example: ‘SL based UE policies for a roamer’: Subscriber has a plan that allows the UE to use a dedicated local S-NSSAI/DNN until the monthly spending limit of $60 is reached. Monthly spending limit of $60 has reached. H-CHF notifies the H-PCF for a UE. H-PCF for a UE removes the URSP rule, which allows the UE to use a dedicated local S-NSSAI/DNN when it roams in the partner PLMN. 23.503 will have to be slightly adjusted to accommodate it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhancing UE policies functionality of R18 SLAMUP in 23.503, such that it will be applicable also in roaming scenario. I.e. Adding roaming functionality to the interaction between PCF and CHF during UE policy association establishment/modification/termination. Aspects of CHF selection are addressed as well. In addition, the text is adjusted to reflect the exact roaming or non-roaming scenarios that are applicable to SM/AM/UE policy respectively. NOTE: this CR is mostly a clean version of S2-2400445r03 from SA2 #160AH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to apply UE policies functionality of R18 SLAMUP in a roaming scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ********************* First Changes *********************

#### 6.1.1.4	Policy decisions based on spending limits

Policy decisions based on spending limits is a functionality that allows PCF taking actions related to the status of policy counters that are maintained in the CHF. The CHF in the non-roaming case or the H-CHF in the Home Routed roaming case provides policy counters for spending limits to the (H-)PCF for the PDU Session. The CHF in the non-roaming case or the H-CHF in the roaming case provides policy counters for spending limits to the (H-)PCF for the UE.

In the non-roaming case this functionality is applicable to session management related policy control, access and mobility management related policy control and UE policy control. In the Home Routed roaming case this functionality is applicable only to session management related policies and UE policies. In the local breakout roaming case this functionality is applicable only to UE policies. The PCF determines to enforce SM, UE and/or AM policy based on subscriber spending limits as indicated by Subscriber spending limits control as described in clause 6.2.1.3, respectively.

NOTE:	In the context of the above function, UE policy control refers to URSP only.

The PCF for the PDU Session and the PCF for the UE use the CHF selection mechanism defined in TS 23.501 [2] to select the CHF that provides policy counters for spending limits.

If the home operator policies indicate that the same CHF shall be selected by the SMF and the (H-)PCF, the (H-)PCF for the PDU Session shall also provide the selected CHF address(es) and if available, the associated CHF instance ID(s) and/or CHF set ID(s) to the SMF in the PDU Session related policy information, otherwise the PCF for the PDU Session does not provide the selected CHF information to the SMF and the SMF applies the CHF selection mechanisms defined in TS 23.501 [2].

If the home operator policies indicate that the same CHF shall be selected by the PCF for the UE and the AMF, then the PCF for the UE shall also provide the selected CHF address(es) and if available, the associated CHF instance ID(s) and/or CHF set ID(s) to the AMF

-	In the non-roaming case, the access and mobility management related policy information or the UE Policy Association supplementary information is used for passing the above information between the PCF for the UE and the AMF.

-	In the roaming case, the UE Policy Association supplementary information is used for passing the above information between the H-PCF for the UE and the AMF via the V-PCF for the UE.

If there is no home operator policy indicating that the same CHF shall be selected by the PCF for the UE and the AMF, then the (H-)PCF for the UE does not provide the selected (H-)CHF information to the AMF and the AMF applies the (H-)CHF selection mechanism defined in TS 23.501 [2].

The identifiers of the policy counters that are relevant for a policy decision in the PCF may be stored in the PCF or possibly in UDR. The PCF is configured with the actions associated with the policy counter status that is received from CHF.

The PCF may retrieve the status of policy counters in the CHF using the Initial or Intermediate Spending Limit Report Retrieval Procedure. The CHF provides the current status of the policy counters to the PCF. The CHF may in addition provide one or more pending statuses for a policy counter together with the time they have to be applied. The PCF shall immediately apply the current status of a policy counter. A pending status of a policy counter shall autonomously become the current status of a policy counter at the PCF when the indicated corresponding time is reached. Subsequently provided information for pending statuses of a policy counter shall overwrite the previously received information.

The PCF may subscribe to spending limit reporting for policy counters from the CHF using the Initial or Intermediate Spending Limit Report Retrieval procedure. If spending limit reporting for a policy counter is enabled, the CHF shall notify the PCF of changes in the status of this policy counter (e.g. daily spending limit of $2 reached or monthly spending limit of $60 is reached) and optionally pending statuses of this policy counter together with their activation time (e.g. due to a billing period that will expire at midnight). The PCF may cancel spending limit reporting for specific policy counter(s) using the Intermediate Spending Limit Report Retrieval procedure, or for all policy counter(s) using the Final Spending Limit Report Retrieval procedure.

The PCF uses the status of each relevant policy counter, and optional pending policy counter statuses if known, as input to its policy decision to apply operator defined actions, e.g.:

-	change the QoS (e.g. downgrade or upgrade Session-AMBR or UE-AMBR), modify the PCC Rules to apply or remove gating or change charging conditions;

-	change the URSP rule (e.g. remove or add an RSD that allows the UE to use a dedicated S-NSSAI and DNN).

The CHF may report to the PCF the removal of the subscriber from the CHF system, and the PCF shall remove all the policy counters of the subscriber accordingly.

If an operator policy indicates that a policy counter and its status should be available for a policy decision before the PCF retrieves the status of the policy counter from the CHF, the PCF stores the policy counter and its status in the UDR at termination of the respective UE Policy Association, AM Policy Association or SM Policy Association.

# ********************* End of Changes *********************