SA WG2 Meeting #163	S2-2407341

Jeju, Korea, May 27-31 2024	was 7113

# Title:	KI#1: conclusions

# Source:	Nokia

# Document for:	Approval

Agenda Item:	FS_VMR_Ph2/19.6

Abstract:	this paper provides conclusions on KI#1 on MWAB architecture

1. Introduction

Based on the outcomes of the moderated email discussion we propose to provide an update to the conclusions for KI#1

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-06.

# Proposed text

## 8.1	KI#1 Conclusion

It is proposed to proceed normatively based on these principles.

1)  The MWAB-gNB indicates to the AMFs it is a MWAB when it establishes the N2 connection to the AMFs.

NOTE 1: This requires RAN3 to define this indication.

2)  If QoS optimization of the N3 connection is required, then the MWAB-gNB is configured (e.g. by OAM) with the mapping of the 5QIs to the DSCP field for TNL used by the UPFs/gNBs in the PLMN/SNPN serving the UE via the MWAB-gNB. This is the used to establish SDFs at the BH-PDU session used for N3 connection(s) based on existing procedures.

# End of proposed text