SA WG2 Meeting #163	S2-2406344r02

May 20 - 31, 2024 Jeju, South Korea	(Revision of S2-240xxxx)

# Source:	China Mobile, ZTE, Huawei, Hisilicon, Nokia, Deutsche Telekom, vivo, NTT DOCOMO, Samsung, Ericsson

# Title:	New WID: System architecture for Next Generation Real time Communication services Phase 2

# Document for:	Approval

Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	System architecture for Next Generation Real time Communication services Phase 2

Acronym:	NG_RTC_Ph2

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 770003 | Study on enhancements to IMS for new real time communication services | Study Item of Stage 1 requirements |
| 790003 | Enhancements to IMS for new real time communication services | Work Item of Stage 1 requirements |
| 850042 | Study on evolution of IMS multimedia telephony service | Study Item of Stage 1 requirements |
| 920036 | Evolution of IMS Multimedia Telephony Service | Work Item of Stage 1 requirements |
| 940066 | Study on system architecture for next generation real time communication services | Study Item of Stage 2 architecture and procedures |
| 970014 | System architecture for Next Generation Real time Communication services | Work Item of Stage 2 architecture and procedures |
| 990023 | CT1 aspects of NG_RTC | Work Item of Stage 3 |
| 990087 | CT4 aspects of NG_RTC | Work Item of Stage 3 |
| 850003 | Study on security support for Next Generation Real Time Communication services | Study Item of Stage 3 |
| 990049 | PS Data Off for IMS Data Channel Service | Work Item of Stage 1 requirements |
| 950005 | Study on Localized Mobile Metaverse Services | Study Item of Stage 1 requirements |
| 1010030 | Study on system architecture for next generation real time communication services phase 2 | Study Item of Stage 2 architecture and procedures Phase 2 |

3	Justification

The system architecture for the next generation real time communication services supporting IMS data channel and AR communication service has been specified in Rel-18.

In SID FS_NG_RTC_Ph2 (ID 1010030), SA2 further studied the system architecture enhancements to support next generation real time communication services, with the following key issues:

- KI#1: Extensible IMS mechanism supporting IMS events in the context of DC communication;

- KI#2: Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session;

- KI#3: Data channel interworking with MTSI UE;

- KI#4: Extensible IMS framework to support authorization and authentication of third-party identities in IMS sessions;

- KI#5: Handling of PS data off exemption for services over IMS DC;

- KI#6: Support of Standalone IMS Data Channel Sessions;

- KI#7: Support multiplexing multiple DC applications over single SCTP connection;

- KI#8: Support of IMS Avatar Communication.

SA2 has concluded the above key issues which was captured in TR 23.700-77.

This work item aims to complete stage 2 normative work based on the conclusions and agreed principles in Rel-19.

4	Objective

The objective of this work item is to enhance IMS network to support the following aspects, based on the conclusions in TR 23.700-77:

WT1. Architectural and procedural enhancement to enable trusted and untrusted DC applications to subscribe to and receive reports about IMS events, including:

-	discovery procedure for event producers, event subscription to the producers and the notification of events;

-	enhancements to services of DCSF, IMS AS, HSS, NEF and MF;

-	definition of events to be reported, including subscriber specific and non-subscriber specific events;

WT2. Enhancements to IMS architecture, interfaces and procedures to expose IMS services in the context of IMS Data Channel

-	support IMS session initiation, update and release with data channel media components by DC applications;

-	enhancements to services of DCSF, IMS AS, NEF and MF;

-	both application and bootstrap data channels shall be supported.

WT3. Enhancements to IMS architecture and procedures to provide interworking with a MTSI UE supporting only legacy media types (audio/video/messaging)

-	procedures to provide media interworking via MF between IMS DC and audio/video/messaging;

-	procedures to report of establishment of an application DC for an application that requires interworking based on solution of KI#1 to DC AS from IMS AS;

-	enhancements to services of IMS AS and MF;

NOTE 1:	Coordination and alignment with SA4 is required.

WT4. Enhancements to the IMS architecture and procedures to support third party identities in IMS sessions

-	mechanism how the serving IMS network authorizes the usage of third party identities;

-	mechanism how IMS network support authentication, authorization, signing and verification of third-party specific identities;

-	storage and retrieval of third party identities;

-	enhancements to IMS subscription data to support third-party identities.

WT5. Enhancements to IMS procedures to support PS Data off for services over IMS DC

-	considering that the services over IMS DC shall be able to be exempted as a whole.

WT6. Enhancements of IMS procedures to support standalone IMS data channel session

-	Scenarios including establishing IMS session with standalone bootstrap DC or a combination of standalone bootstrap DC and application DC, changing an IMS session with audio/video/DC media components to a standalone IMS DC session and adding audio/video media components to a established standalone IMS DC session shall be supported;

WT7. Enhancements of IMS architecture and procedures to support multiplexing and de-multiplexing DC applications over single SCTP connection

-	mechanism to determine whether the UE and the network support multiplexing a SCTP connection for multiple DC applications before media negotiation;

-	functionalities and procedures of multiplexing and de-multiplexing multiple DC applications;

-	enhancements to services of DCSF, IMS AS and MF.

WT8. Enhancements of IMS architecture and procedures to support IMS based Avatar communication

-	definitions of identifiers required for IMS Avatar communication, i.e. Avatar ID and Avatar representation;

-	downloading of list of Avatar ID(s);

-	storage, retrieval, authentication and authorization of Avatar representation;

-	service/capability negotiation between UE and IMS network;

-	functionalities and procedures of supporting media transition and transcoding between a MMTel session using audio/video codec and IMS Avatar based communication, transcoding between speech and gesture (or text) in an IMS Avatar communication.

-	functionalities and procedures to support UE based and network based rendering of IMS Avatar communication;

-	enhancements to services of DCSF, IMS AS and MF.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.228 | Update on IMS architecture and procedures to support Rel-19 features | SA#106 (December 2024) |  |
| TS 23.502 | NEF service enhancements to support event subscription and notification, application initiated IMS data channel session management | SA#106 (December 2024) |  |

6	Work item Rapporteur(s)

Yi Jiang, China Mobile, jiangyi at chinamobile dot com

7	Work item leadership

SA2

8	Aspects that involve other WGs

SA4 for codec, application binding and application synchronization.

9	Supporting Individual Members

Supporting IM name

China Mobile

CATT

Deutsche Telekom

Ericsson

Hisilicon

Huawei

Nokia

NTT DOCOMO

Qualcomm

Samsung

Vivo

ZTE