SA WG2 Meeting #S2-163	S2-2405867

27 - 31 May, 2024, Jeju, South Korea

3GPP TSG-CT WG4 Meeting #122	C4-241527

Changsha, China, 15th – 19th April 2024	was C4-241325

Title:	LS on MPS PDU Session Handling for Non-MPS Subscriber

Response to:

Release:

Work Item:

Source:	CT4

To:	SA2

Cc:

Contact person:	Jones Lu

Jones (dot) Lu (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

1. Overall Description:

In 5G Core Networks, the SBI messages between network functions are handled with different message priorities over HTTP layer, e.g. the messages serving Multimedia Priority Service (MPS) subscribers will be handled with higher priority than non-MPS subscribers.

For a MPS subscriber, the serving AMF and SMF(s) will prioritize the messages over N11 based on the subscription information received from UDM. For a service session involving both MPS subscribers and non-MPS subscriber (e.g. the MPS subscriber makes a voice call to a non-MPS subscriber), the PDU session of the non-MPS subscribers shall also be handled in high priority for end-to-end service assurance. In such a scenario, the SM Policy PCF for the non-MPS subscriber can get MPS priority indication from IMS system and inform the SMF to apply MPS priority for the IMS PDU session temporarily. The SMF then set high priority for SBI service requests towards the AMF. However, if the AMF is not aware about the MPS priority of the PDU session and it does apply normal priority for SBI service request towards the SMF, N11 signalling initiated by the AMF may be discarded by the SMF when overloaded or congested, and the service flow will fail.

CT4 kindly asks SA2 whether there is existing stage 2 procedure to allow the AMF serving non-MPS subscriber also be informed to temporarily apply high priority for the PDU session in above scenario. If not CT4 would like to ask SA2 to define the corresponding stage 2 procedure if possible.

2. Actions:

To SA WG2 group:

ACTION: 	CT4 kindly asks SA2 to evaluate the above scenario and indicate the corresponding stage 2 procedure if exists. Or otherwise define the corresponding stage 2 procedure if possible.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm