SA WG2 Meeting #163  	  S2-2405900

27 May-31 May 2024, Jeju island, Korea

Source:	ZTE

Title:	TR 23.700-84: KI#1_Sol#11 update

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract: This contribution updates Sol#11 of 23.700-84.

# 1. Discussion

This paper proposes to update Sol#11 by adding AoI and some other clarifications in the data collection procedure and remove the following EN:

Editor's note: Whether to perform data collection per UE, per area or both is FFS.

# 2. Proposal

---------- Start of change ----------

6.11  Data collection procedure for LMF-side model training

6.11.1	Functional Description

This solution proposes to reuse the UE positioning procedure defined in TS 23.273 to identify target LMF(s) for measurement data collection. After receiving the data collection request, the LMF(s) can send the measurement data and PRU information to NWDAF for LMF-side model training directly(i.e. without passing GMLC/AMF). In the case of a large amount of measurement data is requested by multiple NWDAFs, sending measurement data directly can avoid overload of AMF/GMLC effectively.

UDM

NWDAF

AMF

LMF

GMLC

6.11.2      Procedures

2.Nudm_UECM_Get Request

1.Measurement data collection request

3.Nudm_UECM_Get Response

4.Measurement data collection request

5.LMF selection

6.Measurement data collection request

7.Measurement data collection response

Figure 6.x.2-1: Procedure of data collection for LMF-side model training

- NWDAF sends measurement data/PRU information collection request to GMLC with target UE ID(s) and/or target AoI and its NF instance ID.

- 2. The GMLC invokes a Nudm_UECM_Get service operation towards the home UDM of the target UE(s).

- 3. The UDM returns the network address (s) of the current serving AMF.

- 4. The GMLC forwards measurement data/PRU information collection request with target UE ID(s) and/or target AoI, NWDAF instance ID to AMF(s).

- 5. The AMF(s) select LMF(s) based on the information as defined in clause 5.1 of TS 23.273 or based on AMF local

- configuration. The selection may use NRF queries.

- 6. The AMF(s) forward measurement data/PRU information collection request to LMF(s) with target UE ID(s) and/or target AoI, NWDAF instance ID.

- 7. LMF(s) collects data from UE/RAN and sends back the measurement data/PRU information of the target UE(s) and/or target AoI.

6.11.3	Impacts on existing services, entities and interfaces

To implement the solution above, the NWDAF shall provide its NF instance ID and target UE ID(s) in the training data collection request. The GMLC shall forward the request to AMF and then AMF shall forward the request to LMF. The LMF needs to prepare training data according to the received target UE ID(s) and send the prepared training data to NWDAF directly.

UE:

- 	Depending on RAN decision, perform and report measurements.

RAN:

- 	Depending on RAN decision, perform and report measurements.

---------- End of changes ----------