SA WG2 Meeting #163	S2-2405910

27 May – 31 May, 2024, Jeju, South Korea

Source:	China Mobile

Title:	Conclusion for Key Issue #1b of Alternative PDU Set QoS Parameters

Document for:	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: This paper proposes conclusions for KI#1b of Alternative PDU Set QoS Parameters.

1	Discussion

This paper proposes conclusion principles for KI#1b of Alternative PDU Set QoS Parameters.

2	Proposal

It is proposed to include the following changes in TR 23.700-70.

* * * * Start of Changes – All New Text * * * *

8.X	Conclusions for Key Issue #1

For key issue #1 of Alternative PDU Set QoS, the following aspects can be included in normative work:

1.	The Alternative QoS profiles shall be enhanced to include PDU Set QoS parameters, e.g. PSDB, PSER.

2.	The AF may provide QoS requirements which include additional one or more Requested Alternative PDU Set QoS Parameter sets (e.g. PSDB, PSER) in a prioritized order, and the corresponding reference to each PDU Set QoS Parameter Set.

3.	PCF generates PCC rule with containing Alternative QoS/PDU Set QoS parameters.

4. 	SMF provides the Alternative QoS Profiles with PDU Set QoS parameters to RAN.

5. 	RAN fulfills the proper Alternative QoS profile with PDU Set QoS parameters.

* * * * End of Changes * * * *