3GPP TSG-WG SA2 Meeting #163	S2-2405919

# Jeju, KR, May 27- May 31, 2024

Source: 	China Mobile, SK Telecom

Title:	Conclusion for KI#3

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS_Ph2 / Rel-19

Abstract: Propose conclusions for KI#3.

1. Discussion

This proposal provides the conclusions for KI#3 as baseline for normative work.

2. Proposal

It is proposed to capture the following changes in TR 23.700-63.

FIRST CHANGE

8.X	Conclusions for Key Issue #3: Study enhancements for UPF handling of headers

The following aspects are concluded as principles for normative work:

-	For the discovery of UPF(s) enhanced to handle header/tags insertion/detection, the following UPF capability is added in UPF profile stored in NRF:

-	Specific traffic detection and handling.

-	For the Reporting Instructions, the AF provides Notification Target Address (+ Notification Correlation ID) attributes to request to receive Session Reports, and direct indication to request that they are sent directly with UPF Exposure Service.

-	The existing functionality for AF traffic influencing is used as baseline for header/tag handling involving a rule in the request.

-	The following rule is provided by the AF:

-	Identifier.

-	Actions.

-	Conditions for actions.

-	Direction of actions.

-	Header/Tag Handling Instructions, including reference and additional data (e.g. active time/period).

-	Reporting Instructions, including type of report, condition for reporting, reporting correlation identifier, and reporting-to-address.

# * * * * End of changes * * * *