SA WG2 Meeting #S2-163	S2-2405944

27 - 31 May, 2024, Jeju, South Korea	(Revision of S2-xx)

Source:	Samsung, Vivo, OPPO, Tencent, Tencent Cloud, InterDigital Inc.

Title:	Conclusions for KI#3

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN /Rel-19

Abstract of the contribution: This contribution proposes conclusions for KI#3.

# Discussion

Based on the discussion on NWM and documented solutions in TR 23.700-84, this contribution proposes the conclusions for KI#3.

# Proposal

It is proposed to adopt the following changes into TR 23.700-84.

*** Start of the change ***

## 8.x	Conclusion for KI#3: NWDAF-assisted policy control and QoS enhancement

Following conclusions are proposed for KI#3:

Based on PCF request, NWDAF may provide assistance information to PCF to assist with QoS determination and modification of QoS parameters at PCF. The PCF may take the assistance information into account to determine the final QoS to be implemented, based on the operator's policy. In addition to provisioning the final QoS to the SMF, the PCF may also provide the final QoS to the NWDAF for generating further optimal QoS.

After the PCF implements the final QoS, the PCF may determine to perform QoS modification based on newly generated assistance information provided by NWDAF. The NWDAF may notify the PCF of the newly generated candidate QoS and other output analytics, if the achieved QoE/ service experience/ performance information of the applied QoS cannot meet the configured threshold.

NOTE 1:	the threshold of QoE/ service experience/ performance information of QoS might be configured by the operator or service provider via reporting threshold.

In detail,

- The PCF may send request to NWDAF for providing QoS and policy assistance information, including analytics ID(s), Filter Information, one or multiple sets of QoS paramters, target QoE/ service experience/ performance information of a service, etc.

NOTE 2:	details of the input data will be discussed in normative stage.

- Based on the PCF request, the QoS and policy assistance information provided by NWDAF may include:

  - one or multiple set(s) of candidate QoS parameters associated with the corresponding priorities and predicted target QoE/ service experience/ performance information.

Editor’s note:	whether to introduce new service or reuse existing service for providing candidate QoS is FFS.

- A combination of (enhanced) existing analytics that are valid during the same validity period, including the Observed service experience, QoS sustainability, UE Communication, Network Performance analytics. The enhancements to the existing analytics may include:

  - enhancing QoS sustainability Analytics to provide finer granulartity QoS KPI, .e.g per UE or per QoS flow level;

  - enhancing UE Communication analytics to provide traffic characterstics (e.g. data rate, data volume, delay, etc.) at application level traffic;

  - enhancing Network Performance analytics to provide gNB resource usage for non-GBR traffic.

Editor’s note:	whether and how to provide PDU Set assistance for XRM-service is FFS.

- Duration and number of usages of QoS Flow.

NOTE 3:	further details of the enhancements to existing analytics will be discussed in normative stage.

NOTE 4:	To provide the assistance for policy control and QoS, NWDAF shall not be aware of PCF internal logic.

*** End of the change ***