SA WG2 Meeting #163	S2-2405996

# 27 – 31 May 2024, Jeju, South Korea

Source: 	NTT DOCOMO

Title: 	KI#4, Interim conclusion

Document for: 	Approval

Agenda Item: 	19.15 (FS_AIML_CN)

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract of the contribution: This contribution proposes interim conclusion for KI#4.

# 1	Discussion

Based on the solutions documented in TR 23.700-84, this paper proposes a set of principles to be considered in the normative work for KI#4.

# 2 Proposal

It is proposed to update TR 23.700-84 according to the following text.

********** First Change (All new text) **********

# 8	Conclusions

## 8.X	Interim Conclusions for Key Issue #4: NWDAF enhancements to support network abnormal behaviours (i.e. signalling storm) mitigation and prevention

It is agreed to consider the following principles for the normative work:

- NWDAF provides new analytics ID (i.e., statistics and prediction) for indication of signalling storm and assistance information for mitigation/prevention of the storm.

- The signalling storm includes the massive requests by UEs and NFs abnormal signaling scenarios.

NOTE 1: The consumers of the analytics ID and the parameters of the analytics request/subscribe (determined by the consumer) are to be specified in the normative work.

- NWDAF may collect the following data to provide the analytics:

-	NF signalling data, collected from NF and/or OAM, considering the standard operational procedures (e.g., data of signallings for UE registration procedure)

-	NF context data, collected from NF and/or OAM, reflecting capacity, load and operational exceptions

-	UE related data, collected from related NFs, e.g., AMF, may cause signaling storm.

NOTE 2: UE related data does not imply data collection from UE.

-	Existing signalling storm related analytics from MDAF and UE related analytics from NWDAF

NOTE 3: The details of the input data for training will be specified in the normative work.

- NWDAF analytics output for indication of signalling storm may include:

-	NF ID which is impacted by the storm

-	NF ID(s) or UE ID(s) which are causing the storm

-	Abnormality ID that identifies the abnormality in the signalling

NOTE 3: The list of the outputs will be finalized in the normative work.

********** End of Changes **********