3GPP TSG-WG SA2 Meeting #163	S2-2406037

# Jeju, South Korea, May 27 - 31, 2024

Source: 	SK Telecom, AT&T, China Mobile, ETRI, KDDI, KPN N.V., Lenovo, LG Uplus, Rakuten Mobile, Samsung, vivo

Title:	Conclusion for KI#4

Document for:	Approval

Agenda Item:	19.15

Work Item / Release:	FS_AIML_CN / Rel-19

Abstract: Propose Evaluation/Conclusion for KI#4.

# 1. Discussion

This proposal proposes the evaluation and conclusion for KI#4 as baseline for normative work.

# 2. Proposal

It is proposed to capture the following changes in TR 23.700-84.

FIRST CHANGE

# 8	Conclusions

## 8.X	Interim Conclusions for Key Issue #4: NWDAF-assisted Network Abnormal Behaviour Mitigation and Prevention

Based on TR 23.700-84 (Use case #3 and #6) and their respective solutions from the 5 approved solutions (#35, #36, #37, #38 and #39), the following aspects are concluded as principles for normative work.

|  | Key Issues |  |  |  | Use cases (optional) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 5 | 6 |
| #35 |  |  |  | X |  |  | X |  |  | X |
| #36 |  |  |  | X |  |  |  |  |  |  |
| #37 |  |  |  | X |  |  |  |  |  | X |
| #38 |  |  |  | X |  |  | X |  |  |  |
| #39 |  |  |  | X |  |  | X |  |  | X |

It is agreed to use Solution #35 and Solution #39 as baseline for normative work.

- NWDAF supports assistance to signalling storm mitigation and prevention by providing detections and predictions on signalling storm caused by massive number of UEs and/or NFs abnormal signalling.

- A new Analytics ID to support Signalling storm Mitigation and Prevention will be defined.

- Analytics Filter include information to assist the NWDAF to identify the signalling storm scenario to detect (e.g. using a specific scenario identifier).

NOTE: 	The consumers of the analytics ID and the parameters of the analytics request/subscribe (determined by the consumer) are to be specified in normative work.

- The NWDAF identifies target NF to obtain data based on the signalling storm scenario included in the analytics request.

- The following Input Data are agreed to be collected from NF, MDAF and/or via NRF/SCP to support generation of the analytics by NWDAF:

- Information related signalling exchange, including

: Request Type of control messages (and related posterior messages), message counts, success and failures, fault information, their req./res. durations from N1/N2 and SBIs as described in clause 6.35.2.2 and table 6.39.1-1.

- Information on NF contexts, including

: UE related number, state transition and timer information of individual UE(s) as described in clause 6.35.2.2 and table 6.39.1-1;

: NF load, status & resource usage as described in table 6.39.1-1.

- UE behaviour related data, including

: output of existing UE related analytics.

- MDAF data, including

: output of existing 5GC Control Plane congestion analysis and/or aggregated data.

NOTE:	The details of input data (including source and detailed data description) are to be specified in normative work.

- In order to reduce signalling overhead, offline bulk data report (clause 6.2.6.1 in TS 23.288 [5]) for UE related data collection may be used.

- The following Output Data from NWDAF are agreed to be included in Analytics output:

- Statistics Analytics, including

: Target NF ID, Abnormality ID along with SUPI/NF lists or a group of UEs

- Prediction Analytics, including

: Confidence level of the prediction

NOTE:	Whether the more IEs are needed will be discussed and specified in normative work.

- The Mitigation or prevention operations are depicted as an example for NFs like table 6.7.5.3-3 in TS 23.288 [5], the final mitigation or prevention operations are based on NF implementation. The mitigation and prevention described in Solution #35 will be used as an example:

- Abnormality ID to recommend actions of NFs, based on operator policy including

: UE-related configurations (back-off timers, thresholds, slice or priority based overload control, etc)

: NF-related configuration ((re)discovery/selection, prioritizations, etc)

NOTE:	Examples of prevention/mitigation actions based on NWDAF analytics on signaling storm prediction and/or detection may be further defined during the course of the normative work.

Editor's note:	It is FFS whether SCP entity itself can be a source of input data.

Editor's note:	It is FFS whether new service/logic function for providing recommendation from NWDAF to service consumer for mitigating and preventing signalling storm.

# * * * * End of changes * * * *