3GPP TSG-WG SA2 Meeting #163	S2-2406099

Jeju, Korea, May 27th – 31st, 2024

Source:	Huawei, HiSilicon

Title:	KI#3: Evaluation and Conclusion

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract: This document provides the evaluation and conclusion for KI#3 regarding the control of UE's access to 5GS via a wireless access backhaul.

1. Discussion

This document tries to add the evaluation and conclusion for KI#3.

KI#3 in TS 23.700-06 is for controlling of UE's access to 5GS via a wireless access backhaul:

5.3 Key Issue #3: Control of UE's access to 5GS via a wireless access backhaul This key issue is to investigate efficient control of UE access to MWAB. In particular, the following aspects should be addressed: - Whether and how to enhance the existing CAG mechanism to control and manage the access of a UE via MWAB. NOTE 1: Support of legacy UE(s) shall be considered. NOTE 2: Aspects related to RAN need to be coordinated with RAN WGs.

By now there are three solutions related to KI#3, namely:

- Solution #1: Architecture enhancements to support MWAB operations;

- Solution #6: Reusing CAG mechanism for managing UE's access to MWAB-gNB;

- Solution #7: Architecture enhancement to support MWAB-gNB for SNPN;

Solution #1 proposes to reuse the mechanism defined for Rel-18 VMR, i.e., for CAG capable UE, CAG ID can be reused for control the access to the MWAB, if the MWAB is configured by OAM of PLMN 2 to broadcast CAGs.

Solution #6 proposes to reuse the existing CAG mechanism defined by NPN for managing UE's access to MWAB-gNB.

Solution #7 suggests to use a dedicated SNPN ID to control the UE to access the MWAB-gNB.

In addition, based on the moderated email discussion, it seems that:

-	"Most company agreed to reuse the existing CAG mechanism (as defined in Rel-18) as an optional method for managing UE’s access to MWAB-gNB."

-	"Most companies agreed that existing SNPN control mechanism to manage the UE’s access to MWAB-gNB, in case it is serving a SNPN."

Observation 1: There is no major comments on reusing the existing NPN mechanisms to manage the UE’s access to MWAB-gNB.

Proposal 1: it is proposed to reuse existing CAG mechanism (as defined in Rel-18) as an optional method for managing UE’s access to MWAB-gNB, in case it is serving a PNI-NPN.

Proposal 2: it is proposed to reuse existing SNPN control mechanism to manage the UE’s access to MWAB-gNB, in case it is serving a SNPN.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-06.

# * * * * First change * * * *

7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

7.3	KI#3 Evaluation

By now there are three solutions related to KI#3, namely:

-	Solution #1: Architecture enhancements to support MWAB operations;

-	Solution #6: Reusing CAG mechanism for managing UE's access to MWAB-gNB;

-	Solution #7: Architecture enhancement to support MWAB-gNB for SNPN;

Solution #1 proposes to reuse the mechanism defined for Rel-18 VMR, i.e., for CAG capable UE, CAG ID can be reused for control the access to the MWAB, if the MWAB is configured by OAM of PLMN 2 to broadcast CAGs.

Solution #6 proposes to reuse the existing CAG mechanism defined by NPN for managing UE's access to MWAB-gNB.

Solution #7 suggests to use a dedicated SNPN ID to control the UE to access the MWAB-gNB.

Table 7.3-1:  Summary of the solutions of KI#3

| Solutions | Requirements on MWAB-gNB | Controlling of the access of UE | Impact to current procedures |
| --- | --- | --- | --- |
| Solution #1 | (Existing mechanism) MWAB is configured by its serving PLMN OAM to broadcast CAGs. | (Existing mechanism) UE is CAG capable: using CAG to control; | No. |
| Solution #6 | (Existing mechanism) MWAB is configured either during the communication with its serving PLMN OAM of or (pre-)configuration mechanism. | (Existing mechanism) UE is CAG capable: using CAG to control; UE is not CAG capable: other existing mechanisms e.g., forbidden Tracking Area; | No. |
| Solution #7 | (Existing mechanism) MWAB is configured List of NIDs per PLMN ID (and others as in clause 5.30.2.2 of TS 23.501 [2]) | (Existing mechanism) UE is SNPN capable: using SNPN ID; | No. |

Note that all candidate solutions are considered as free of procedure impact.

# * * * * Second change * * * *

8.3	KI#3 Conclusion

The following conclusions are reached as baseline for normative work to support the control of UE's access to 5GS via a wireless access backhaul:

-	In case if MWAB-gNB is serving a PNI-NPN, the existing CAG mechanism (as defined in Rel-18) as an optional method is reused to manage the UE's access to MWAB-gNB.

-	In case if MWAB-gNB is serving an SNPN, the existing SNPN control mechanism is reused to manage the UE's access to MWAB-gNB.

-	SNPN ID and GIN can be used to control the access of the UE in mixed gNB and MWAB-gNB cases;

-	In case the UE does not support functionalities regarding PNI-NPN and SNPN, or PNI-NPN/SNPN are not deployed by the network, the NG-RAN and 5GC are allowed to use other existing mechanisms e.g., forbidden Tracking Area, to manage its access control to MWAB-gNB.

# * * * * End of changes * * * *