SA WG2 Meeting #163	S2-2406145

Jeju, Korea, May 27 – May 31, 2024

Source:	OPPO

Title:	KI#1, Conclusions for KI#1

Document for:	Approval

Agenda Item:	19.7

Work Item / Release:	FS_5G_ProSe_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes KI#1 Conclusions.

- Discussion

After the NWM discussion, this contribution proposes KI#1 Conclusions.

2.	Text proposal

It is proposed to agree the following changes to TR 23.700-03:

* * * 1st  change (all new text)* * *

8.X	Key Issue #1 Conclusions

For key issue #1, the following are taken as conclusions.

-	For 5G ProSe multi-hop UE-to-Network Relay Discovery, both Model A and Model B are supported.

-	Hop-Limit IE is used for controlling the maximum hops and is provided to both the 5G ProSe Remote UE and UE-to-Network Relays per RSC using the Policy/Parameter provisioning as described in clause 5.1.4 of TS 23.304 [4].

-	Intermediate UE-to-Network Relay includes its own User Info ID when relaying the Model A or Model B discovery message.

-	Hop-Count IE and Accumulated QoS for PC5 link IE may be added in the Model A and Model B discovery messages as describe in the solution#1.

-	Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay.

-	Remote UE sends the selected path information as describe in the solution#2 to the Intermediate UE-to-Network Relay for communication setup.

* * * End of change * * *