SA WG2 Meeting #163	S2-2406146

Jeju, Korea, May 27 – May 31, 2024

Source:	OPPO

Title:	KI#2, Conclusions for KI#2

Document for:	Approval

Agenda Item:	19.7

Work Item / Release:	FS_5G_ProSe_Ph3 / Rel-19

Abstract of the contribution: This contribution proposes KI#2 Conclusions.

- Discussion

After the NWM discussion, this contribution proposes KI#2 Conclusions.

2.	Text proposal

It is proposed to agree the following changes to TR 23.700-03:

* * * 1st  change (all new text)* * *

8.X	Key Issue #2 Conclusions

For key issue #2, the following are taken as conclusions.

-	For 5G ProSe multi-hop UE-to-UE Relay Discovery, both Model A and Model B are supported.

-	Hop-Limit IE is used for controlling the maximum hops and is provided to both the 5G ProSe End UEs and UE-to-UE Relays per RSC using the Policy/Parameter provisioning as described in clause 5.1.5 of TS 23.304 [4].

-	UE-to-UE Relay includes its own User Info ID when relaying the Model A or Model B discovery message.

-	5G ProSe End UE selects the multi-hop UE-to-UE Relay path to another 5G ProSe End UE.

-	5G ProSe End UE sends the selected path information as describe in the solution#6 to the UE-to-UE Relay for communication setup.

* * * End of change * * *